Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: REINFORCED CONCRETE STRUCTURES – II  		      	       Time: 3 hours

Subject Code: 09CE217			       					             	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

Use of IS 456-2000,IRC codes, IS 3370, IS 1893-2002, IS13920-1994 and Design Aids (SP16) permitted. Use M20 concrete and Fe415 steel unless otherwise specified.

1.	The tread is always _______ rise.

2.	The shear key is needed in the retaining wall, when _______.

3.	Cracking tensile stress for water tank may be checked by the formulae _______.

4.	The minimum reinforcement for water tank in each direction is _______.

5.	The impact load for the span of 5m of tracked vehicles is ______ of the applied live load.

6.	Courbon’s method is used for the design of _______ and Pigeaud’s chart is used for the design of _______.

7.	The exterior columns are assumed to resist _______ the horizontal shear taken by the interior column.

8.	For the analysis of horizontal load like wind load _______ method is commonly used.

9.	Along the support _______ yield line is developed.

10.	The ultimate load for the isotropically reinforced square slab simply supported and uniformly 	loaded is _______.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw a typical cross section of a cantilever retaining wall and explain its components.

12.	List out the load acting on an underground water tank stating its critical cases.

13.	What are the types of loads considered for bridge design indicating its application?

14.	Give the general principle behind seismic design.

15.	State the assumption made in Yield Line Theory.

PART – C  (5 x 15 = 75 MARKS)

16.	Design a dog-legged stair for a building in which the vertical distance between floors is 3.2m. The stair hall measures 2.5m x 5m. The live load may be taken as 2.5kN/m3. Use M20 concrete and Fe415 steel bars.

(OR)

17.	Design the stem portion of a cantilever retaining wall to retain an earth embankment of 4 m 	height above ground level. The density of earth is 18 kN/cu.m and its angle of repose is 30 	degrees. The embankment is horizontal at its top. The SBC of soil is 200 kN / sq.m. Assume 	any other data required. Use M20 grade concrete and Fe 415 steel. Also perform the stability 	check.

18.	An RCC water tank resting on ground is 3.65m x 2.16m with a depth of 2.1m.  Use M20 	concrete and Fe415 steel. Design the tank walls.

(OR)

19.	Explain the step by step procedure of the design of a circular water tank for:

a.	Flexible base   	b.	Fixed base.

[P.T.O]

20.	Design a deck slab for the slab bridge with the following data:

Carriage way	- Two lane (7.5m wide)    	Foot paths - 1m on either side

Clear span 	-  5.5m                             	Wearing coat - 100mm

M25 grade concrete and Fe415 steel are used. Design the bridge for class AA loading.

(OR)

21.	a.	Give an account of T beam Girder Bridge and the methods available for design.		(5)

b.	Explain in detail the Courbon’s method. 										(10)

22.	A three bay building frame has three storeys, each 4m, with bay dimensions 6m, 3m and 6m;

The spacing between one frame to other is 3m interval. Thickness of floor slab is 100mm. Live load is 2 kN/sq.m. Floor Finish = 0.5 kN/sq.m. Size of the beam is 200mm x 400mm. Size of the column is same as that of beam.  Analyse the building frame for maximum positive bending moment in beams by considering the substitute frame in the second storey.

(OR)

23.	a.	Discuss in detail the method of wind load analysis in frames. 		          			(7.5)

b.	Explain with neat sketches the ductile detailing of moment resisting frames.    		(7.5)

24.	Explain the analysis of slab in detail. Analyse a simply supported square slab by virtual method.

(OR)

25.	A reinforced concrete slab  5 x 5m is simply supported along the four edges and it is reinforced with 10mm dia M.S rod at 150mmc/c in both ways. The average effective depth is 100mm. The overall depth is 130mm. The slab carrier flooring is 50mm thick having unit weight =2.2kN/m. Determine the max permissible service load.