Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DESIGN OF STEEL STRUCTURES (LIMIT STATE METHOD)   														       						      	       Time: 3 hours

Subject Code: 09CE219				       						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Mention any three rolled steel sections.

2. 	What are the types of connections used in steel structures?

3. 	Define Tension Members.

4. 	What do you mean by laced columns?

5. 	How can strain hardening be used in the field?

6. 	What are the possible failures of beams?

7. 	What is the necessity of plate girders?

8. 	What is the use of column bases?

9. 	What is the minimum pitch of a truss for a GI sheet covering?

10. 	Write any three types of roof trusses.

PART – B (5 x 3 = 15 MARKS)

11. 	What are the advantages of welded connections?

12. 	Explain lug angle.

13. 	What is the use of built-up beam sections?

14. 	Explain gusseted base.

15.  	How will you calculate the design wind speed?

PART – C (5 x 15 = 75 MARKS)

16. 	Design a lap joint between the two plates each of width 120mm, if the thickness of one plate is 16mm and the other is 12mm. The joint has to transfer a design load of 160kN. The plates are of Fe410 grade. Use bearing type bolts.

(OR)

17. 	Design a welded connection to connect two plates of width 200mm and thickness 10mm for 100 percent efficiency.

18. 	Design a single angle section for a tension member of a roof truss to carry a factored tensile force of 225kN. The member is subjected to the possible reversal of stress due to the action of wind. The length of the member is 3m. Use 20mm shop bolts of grade 4.6 for the connection.

(OR)

19. 	A column 4m long has to support a factored load of 6000kN. The column is effectively held at both ends and restrained in direction at one of the ends. Design the column using beam sections and plates.

20. 	A simply supported steel joist of 4m effective span is laterally supported throughout. It carries a total uniformly distributed load of 40kN (inclusive of self weight). Design an appropriate section using steel of grade Fe410.

(OR)

[P.T.O]

21. 	An ISMB 400 beam is to be connected to an ISHB250@537N/m to transfer an end force of 140kN. Design a double plated welded connection.

22. 	Design a welded plate girder of span 24m to carry on a superimposed load of 35kN/m. Avoid use of bearing and intermediate stiffeners.

(OR)

23. 	Design a slab base for a column ISHB 350@ 577N/m carrying an axial factored load of 1000kN. M20 concrete is used for the foundation. Provide welded connection between column and base plate.

24. 	Symmetric trusses of span 20m and height 5m are spaced at 4.5m centre to centre. Design channel section purlins to be placed at suitable distances to resist the following loads.

Weight of sheeting including bolts = 171kN/m2

Live load = 0.4 kN/m2

Wind load = 1.2kN/m2, suction

Spacing of purlins = 1.4m

(OR)

25. 	A power plant structure having maximum dimension more than 60m is proposed to be built on a downhill side near Dehradun. The height of the hill is 400m with a slope of 1in 3. If the location is 250m from the crest of the hill on downward slope and its eve board is at a height of 9m, determine the design wind pressure.