Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: FOUNDATION ENGINEERING   						      	       Time: 3 hours

Subject Code: 09CE221						       	       		       	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is a pressure bulb?

2.	Define Settlement.

3.	What is a sampler?

4.	Borings are ___________ methods.

5.	When do you provide a mat foundation?

6.	What is a pile?

7.	What are the two classical earth pressure theories?

8.	What are the classifications of sheet pile wall?

9.	Define compaction.

10.	Define Grouting.

PART – B  (5 x 3 = 15 MARKS)

11.	Bring out the effect of water table on bearing capacity.

12.	What do you mean by Percussion drilling?

13.	What is Negative Skin Friction?

14.	Differentiate between Active and Passive Earth Pressure.

15.	What is soil stabilization? What are its uses?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive as per Boussinesq’s theory, the expression for vertical stress at any point in a soil mass due to point load, and state the assumptions.

(OR)

17.	Compute the safe bearing capacity of a square footing 1.5 m x1.5m, located at a  depth of 1 m below the ground level  in a soil of average density 20 kN/ m3. ф = 200 , NC=17.7, Nq = 7.4 and Nγ =5.0. Assume a suitable factor of safety and that the water table is very deep. Also compute the reduction in safe bearing capacity of footing if water table rises to the ground level.

18.	Explain the Geophysical Methods of Soil Exploration.

(OR)

19.	Discuss the Standard Penetration Test with corrections.

20.	Explain the classification of Piles based on different criteria with sketches.

(OR)

21.	What is a Cassion? What are the classifications of caissons? Describe the component parts of a pneumatic caisson with a neat sketch.

22.	A Gravity Retaining wall retains 12 m of a backfill, γ =17.7 kN/ m3, ф = 250 with a uniform horizontal surface. Assume the wall interface to be vertical, determine the magnitude and point of application of total active pressure. If the water table is at a height of 6 m, how far do the magnitude and the point of application of active pressure change?  Draw the pressure diagram.																		           [P.T.O]

(OR)

23.	A masonry retaining wall is 1.5 m wide at the top, 3.5 m wide at the base and 6 m high. It is trapezoidal in section and has a vertical face on the earth side. The back fill is level with the top. The unit weight of the fill is 16 kN/ m3 for the top 3 m and 23 kN/ m3 for the rest of the depth. The unit weight of masonry is 23 kN/ m3. Determine the total lateral pressure on the wall per meter run and the maximum and minimum pressure intensities of normal pressure at the base. Assume ф = 300 for both grades of soil.

24.	The following data have been obtained in a standard laboratory proctor compaction test on glacial till:

| Water content % | 5.02 | 8.81 | 11.25 | 13.05 | 14.40 | 19.25 |
| --- | --- | --- | --- | --- | --- | --- |
| Weight of container & compacted soil (N) | 35.80 | 37.30 | 39.32 | 40.00 | 40.07 | 39.07 |

The specific gravity of the soil particles is 2.77. The container is 9.44cm3 in volume and its weight is 19.78 N. Plot the compaction curve and determines the optimum moisture content. Also compute the void ratio and degree of saturation at optimum condition.

(OR)

25.	Write short notes on

a.	Mechanical Stabilization.

b.	Lime Stabilization.

c.	Cement Stabilization.