Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: FLUID MECHANICS AND MACHINERY				       	       Time: 3 hours

Subject Code: 09CE242			       					             	      	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define surface tension.

2.	Define capillarity.

3.	Define stream line.

4.	What is meant by velocity potential function?

5.	State the use of a pitot tube.

6.	State the use of a venturimeter.

7.	Give an example of a positive displacement pump.

8.	What is meant by cavitation?

9.	Define specific speed of a turbine.

10   Give an example of an impulse turbine.

PART – B (5 x 3 = 15 MARKS)

11.	What do you mean by total pressure and centre of pressure?

12. 	State the assumptions made in Bernoulli’s equation.

13.	What is meant by a water hammer?

14. 	Name the factors that affect the selection of a pump.

14.	State the functions of a draft tube.

PART – C (5 x 15 = 75 MARKS)

16.	a. 	Calculate the capillary rise in a glass tube of 25mm diameter when immersed,

i.	vertically in water

ii.	at 1300 in mercury. The surface tension of water and mercury 	are 	0.0725 N/m and 0.52 N/m respectively.                                                                         			(9)

b. 	Calculate the specific weight, density and specific gravity of one liter of a liquid which         weighs 7N.									  								(6)

(OR)

17. 	a. 	Write a short note on various types of mechanical gauges used for pressure measurement.

(6)

b. 	Determine the total pressure on a circular plate of diameter 1.5m which is placed vertically 		in water in such a way that the centre of plate is 3m below the free surface of water. Find 			the position of centre of pressure also.                                                               		(9)

18. 	Water is flowing through a taper pipe of length 100m having diameter 600mm at the upper end and 300mm at lower end at the rate of 50 lit/sec. The pipe has a slope of 1 in 30. Find the pressure at the lower end if the pressure at the higher end is 19.62 N/cm2.

(OR)

19. 	a. 	Differentiate between free and forced vortex flow.                                     			(3)

b. 	If, for a two dimensional potential flow the velocity potential is given by x(2y-1),  determine the velocity at point P (4,5). Determine the value of stream function at point P.

(12)

[P.T.O]

20. 	a.	State the advantages of a triangular notch over a rectangular notch.               			(3)

b. 	Find the discharge through a trapezoidal notch which is 1m wide at top and 0.4m at bottom and its height is 30 cm.  The total head of water is 20cm. Assume the coefficient of discharge for triangular and rectangular notch as 0.6 and 0.62 respectively.       		(12)

(OR)

21. 	A syphon of diameter 200mm connects two reservoirs having difference in elevation of 20m. The length of the syphon is 500m and the summit is 3m above water level in the upper reservoir. The length of the pipe from upper reservoir to summit is 100m. Determine the discharge through the syphon and also the pressure at the summit. Neglect minor losses. The coefficient of friction f= 0.005.

22. 	The outer diameter of an impeller of a centrifugal pump is 400mm and outlet width 50mm. The pump is running at 800rpm and is working against a total head of 15m. The vane angle at outlet is 40o and manometer efficiency is 75%. Determine the velocity of flow at the outlet, velocity of water leaving the vane, angle made by absolute velocity at the outlet with the direction of motion at the outlet and discharge.

(OR)

23. 	A double acting reciprocating pump running at 40 rpm is discharging 1 m3 of water per minute. The diameter of the piston is 200mm. The pump has a stroke of 400mm. The delivery and suction heads are 20m and 5m respectively. Find the slip of the pump and power required to drive the pump.

24. 	A three jet pelton wheel turbine is required to generate 10,000 KW under a net head of 400m. The blade angle at outlet is 15o and the reduction in relative velocity while passing over the blade is 5%. If the overall efficiency of the wheel is 80%, Cv = 0.98 and speed ratio = 0.46, find the diameter of the jet, total flow and force exerted by the jet on the buckets.

(OR)

25. 	Explain in detail the working principle of a Kaplan turbine and a Submersible pump.