Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MECHANICS OF SOLIDS  								       Time: 3 hours

Subject Code: 09CE245			       				               	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Engineering stress and strain are calculated using the _______cross-sectional area and gauge length of the specimen.

2. 	The relationship between modulus of rigidity G, and modulus of elasticity E is ________.

3. 	A simply supported beam has a uniformly distributed load on it. The bending moment diagram will be a _______.

4. 	Under what circumstances is Macaulay’s method very useful?

5. 	Define neutral axis of a beam.

6. 	What is a flitched beam?

7. 	Define Principal plane.

8. 	Define Distortion Energy Theory.

9. 	Where will be the maximum slope and deflection of a cantilever beam?

10. 	Define slenderness ratio.

PART – B (5 x 3 = 15 MARKS)

11.	A cylindrical pipe of diameter 1.5 m and a thickness 1.5 cm is subjected to an internal fluid pressure 1.2 N/mm2. Find the hoop stress and longitudinal stress developed in the pipe.

12. 	What is the least internal radius to which a bar of steel 100 mm wide and 10 mm thick can be bent so that the maximum bending stress will not exceed 500 N/mm2? Assume E = 2 x 105 N/mm2.

13. 	What are the assumptions made in the theory of simple bending?

14. 	A rectangular bar of cross sectional area of 11000 mm2 is subjected to a tensile load P as shown in the figure below. The permissible normal and shear stresses on the oblique plane BC are given as 7 N/mm2 and 3.5 N/mm2 respectively. Determine the safe value of P.

15. State the moment area theorems for slope and deflection.

PART – C (5 x 15 = 75 MARKS)

16.	A circular bar ABCD is rigidly fixed at A and D and is subjected to axial forces as shown in the figure below. Determine the reactions and forces in each portion of the bar and the displacement of point B and C. Take E = 200 kN/mm2.

[P.T.O]

(OR)

17. 	A composite bar made up of aluminium and steel is held between two supports as shown in the figure below. The bars are stress free at a temperature of 42C. What will be the stresses in the two bars when the temperature drops to 24C, if:

a.	the supports are un-yielding,

b.	the supports come nearer to each other by 0.1 mm. The cross sectional area of the steel bar is 160 mm2 and that of aluminium bar is 240 mm2. Take E for aluminium as 0.7 x 105 N/mm2 and that for steel as 2 x 105 N/mm2. The coefficients of thermal expansion for aluminium and steel are 24 x 10-6 per C and 12 x 10-6 per C respectively.

18.  	A cantilever beam of length 5 m is loaded as shown in the figure below. Draw the S.F. and B.M. diagrams for the cantilever.

(OR)

19.  	Draw the shear force and bending moment diagrams for the overhanging beam carrying uniformly distributed load of 2 kN/m over the entire length and a point load of 2 kN as shown in the figure below. Also locate the point of contraflexure.

20. 	A timber beam of rectangular section is to support a load of 20 kN uniformly distributed over a span of 3.6 m when the beam is simply supported. If the depth of section is to be twice the breadth, and the stress in the timber is not to exceed 7 N/mm2, find the dimensions of the cross section.

How would you modify the cross section of the beam, if it carries a concentrated load of 20 kN placed at the centre with the same ratio of breath to depth?

(OR)

21.  	A closed coiled helical spring has a stiffness of 10 N/mm. Its length when fully compressed with adjacent coils touching each other is 400 mm. Taking C = 8 x 104 N/mm2,

a.	Determine the wire diameter and mean coil diameter if their ratio is 1/10.

b.	If the gap between any two adjacent coils is 2 mm, what maximum load can be applied 	before the spring becomes solid i.e. adjacent coils touch each other?

c.	What is the corresponding maximum shear stress in the spring?				    [P.T.O]

22.  The intensity of resultant stress on a plane AB at a point in a material under stress is 800 N/cm2 	and it is inclined at 30 to the normal to that plane. The normal component of stress on another 	plane BC at right angles to plane AB is 600 N/cm2. Determine:

a.	the resultant stress on the plane BC.

b.	the principal stresses and their directions.

c.	the maximum shear stresses and their planes.

(OR)

23. 	The load on a bolt consists of an axial pull of 15 kN together with a transverse shear of 7.5 kN. Determine the diameter of the bolt according to:

a.	maximum principal stress theory	 	b.	maximum shear stress theory

c.	maximum strain theory				d.	strain energy theory

e.	shear strain energy theory.

Elastic limit in tension is 285 N/mm2, and a factor of safety of 3 is to be applied. Take  = 0.3.

24. 	A simply supported beam of length 6 m carries an eccentric point load of 50 kN at a distance   4 m from the left end. Determine:

a.	the deflection under the point load

b.	maximum deflection.

Take E= 2 x 105 N/mm2 and I = 1 x 108 mm4.

(OR)

25. 	A hollow cylindrical cast iron column is 4 m long with both ends fixed. Determine the minimum diameter of the column if it has to carry a safe load of 250 kN with a factor of safety of 5. Take the internal diameter as 0.8 times the external diameter. Take c = 550 N/mm2 and

a = 1 / 1600 in Rankine’s formula.