# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: STABILITY OF STRUCTURES                			       		Time: 3 hours

# Subject Code: 09CE306				         		        		 	Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Explain equilibrium approach with an example.									(5)

b.	Explain the stability of link models.												(5)

c.	Obtain the critical load of a hinged column subjected to an axial load P using energy 	approach.  The moment of inertia is linearly varying from 2I0 at supports to 4I0 at mid 	span.																	      (10)

2.	a.	Write short notes on Shanley’s model.											(8)	b.	Explain the procedure of finding double modulus.							     (12)

(OR)

3.	a.	Determine the maximum deflection and maximum moment in a simply supported beam 		of length ‘l’ that is simultaneously acted on by a transverse load ‘P’ which acts at mid 		span and an axial force ‘P’.  Assume that the material obeys Hooke’s law.		      (15)

b.	Explain stability functions.													(5)

4.	Explain lateral buckling of thin walled open sections in pure bending with an example.

(OR)

5.	Describe torsional flexural buckling using energy method with an example.

6.	a.	Explain governing differential equation of a plate.								(5)

b.	Give notes on post buckling strength of plates.								     (15)

(OR)

7.	A rectangular plate (3a x 4a) is simply supported along all the edges and is compressed bi- axially along the shorter edges with a force of ‘Nx’ N/m and along the longer edges with a force of ‘Nx’ N/m.  Determine the critical value of ‘Nx’ using finite difference method.

8.	a.	Find the critical load of an axially loaded cylinder using Donnel’s equation.		      (10)

b.	Determine the critical stress for a cylindrical shell that is simply supported at its ends and subjected to a uniform lateral pressure.										      (10)

(OR)

9.	a.	Differentiate symmetric and asymmetric bifurcation.							      (10)

b.	Explain path tracing and path switching.										      (10)