Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: COMMUNICATION ENGINEERING					      	      Time: 3 hours

Subject Code: 09EC204							      			      	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Modulation.

2.	Define frequency.

3.	Give formula for signaling rate in PCM.

4.	Give one advantage of delta modulation.

5.	What is the bandwidth of single channel in GSM?

6.	What is the use of GMSC in GSM network?

7.	What is Geo Stationary satellite?

8.	What are the basic components of a satellite earth station subsystem?

9.	How is index profile for the fiber produced?

10.	What are the advantages of fiber systems?

PART – B  (5 x 3 = 15 MARKS)

11.	Define AM and draw its spectrum.

12.	Twelve different message signals, each of bandwidth 10 kHz are to be multiplexed and transmitted. Determine the minimum bandwidth required for PAM/TDM System.

13.	What is the use of black list in GSM network?

14.	Define Kepler’s Law.

15.	What are the different modes of propagation of light?

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the bandwidth occupied by a sinusoidally frequency modulated carrier for which the modulation index is 2.4 and the maximum frequency is 15 kHz.

(OR)

17.	A telephone transmitter using AM has unmodulated carrier output power of 20 kW and can be modulated to a maximum depth of 80% by a sinusoidal modulating voltage without causing overloading. Find the value to which unmodulated carrier power may be increased without resulting in overloading if the maximum permitted modulation index is restricted to 60%.

18.	Bandwidth of input to a pulse code modulator is restricted to 4 kHz. The signal varies from       –3.8 V to 3.8 V and has the average power of 30 mW. The required signal to quantization noise power ratio is 20 dB. The modulator produces binary output. Find:

a.	No. of bits required per sample.

b.	Output of 30 such PCM codes is time multiplexed. What is the minimum required transmission bandwidth for the multiplexed signal?

(OR)

19.	Explain the generation and noise performance of delta modulation.

20.	Describe the GSM architecture in detail.

(OR)

[P.T.O]

21.	Write in detail about cellular engineering concept.

22.	Discuss the principle of satellite communication with a diagram.

(OR)

23.	The INTELSAT IV satellite operates in the bands 5932-6418 MHz and 3707-4193 MHz. The satellite uses 12 transponders, each of bandwidth 36 MHz. An 8 MHz guard band separates the transponder channels into two groups of six and guard band between transponder channels in each group of six is 4 MHz. Draw the frequency channeling scheme and  indicate which are the uplink and which are the downlink assignments.

24.	How can one determine the number of propagated modes in step index fibers?

(OR)

25.	With a circuit diagram discuss the principle of operation of fiber optic communication system.