Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: LINEAR INTEGRATED CIRCUITS AND APPLICATIONS	       Time: 3 hours

Subject Code: 09EC220			       						              	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Name the technology used for the fabrication of ICs.

2.	Which diode is clamped between base and collector of a transistor to avoid saturation?

3.	Define CMRR.

4.	Mention two applications of OP-AMPs.

5.	What do you mean by Schmitt trigger?

6.	What do you mean by voltage regulator?

7.	What is the roll-off rate of a 1st order filter?

8.	Draw the pin configuration of IC 555.

9.	Give one application of PLL.

10.	Which is the fastest ADC?

PART – B (5 x 3 = 15 MARKS)

11.	List the advantages of integrated circuit over discrete component circuit.

12.	List six characteristics of an ideal OP-AMP.

13.	Draw the circuit of a monostable multivibrator using IC 741.

14.	Draw the circuit diagram of a generalized second order filter.

15.	Give the important specifications of D/A and A/D convertors.

PART – C (5 x 15 = 75 MARKS)

16. 	Explain the following:

a.	Oxidation.				b.	Photolithography.				c.	Diffusion.

(OR)

17. 	Explain the following:

a.	Junction capacitor.		b.	Thin film Resistor.			c.	Epitaxial Resistor.

18.	a.	Define slew rate.  How is the slew rate measured?								(7)

b.	Draw the circuit of a log amplifier using two OP-AMPs and explain its operation.	(8)

(OR)

19. 	Discuss the design of Inverting and Non-inverting amplifiers with suitable circuits. Sketch its input and output waveform.

20.	Explain the operation of a square wave generator by drawing the capacitor and output voltage waveform.

(OR)

21. 	a.	Draw and explain the functional diagram of a 723 regulator.						(10)

b.	How is current boosting achieved in a 723 IC?									(5)

22.	Design a fourth order Butterworth Low Pass Filter having an upper cutoff frequency of 1 KHz.

(OR)

23. 		Draw and explain the functional diagram of a 555 timer.

[P.T.O]

24.	a.	Draw the circuit of a PLL and show how a PLL is used as an AM detector.			(10)

b.	Define capture range, lock in range and pull-in-time.								(5)

(OR)

25. 	 Explain the operation of a Dual slope ADC.