Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MICROWAVE AND OPTICAL COMMUNICATION ENGINEERING														      					      	       Time: 3 hours

Subject Code: 09EC223			       						                     Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a waveguide choke flange?

2. 	Why is a magic tee referred to as an E-H tee?

3. 	What is drift space?

4. 	The condition for oscillation in a reflex klystron is__________.

5. 	BARITT is an acronym for __________.

6. 	What is negative resistance?

7. 	Numerical aperture is used to describe the __________ of an optical fiber.

8. 	The expression for critical angle is __________.

9. 	What is a heterojunction?

10. 	An optical detector converts __________ into __________.

PART – B  (5 x 3 = 15 MARKS)

11.	List the advantages of microwaves.

12. 	Briefly explain the applegate diagram.

13. 	Define insertion loss.

14. 	State Snell’s law.

15. 	A photodiode is constructed of GaAs, which has a band gap of 1.53eV at 300K. Find the 	cut-	off wavelength

PART – C  (5 x 15 = 75 MARKS)

16. 	Write a note on:

a.	waveguide corners.        	b.	bends.           	c.	twists.      						(3x5)

(OR)

17. 	Derive the scattering matrix for a directional coupler.

18. 	Describe the working of a TWT amplifier with a neat diagram.

(OR)

19. 	Derive the Hull-cutoff voltage equation for magnetron oscillators.

20. 	Explain the construction, principle of operation and applications of TRAPAT diodes.

(OR)

21. 	Explain about the measurement of VSWR with a neat diagram.

22. 	Discuss the following types of fibers:

a.	Single mode step index fiber.  		b.	Multimode step index fiber.                                  	c.	Multimode graded index fiber.                            									(3x5)

(OR)

[P.T.O]

23.	Write a note on:

a.	Attenuation.       		b.	Dispersion.		c.	Polarization.          				(3x5)

24. 	Describe the construction and working of an LED.

(OR)

25. 	Explain PIN photo detector with a neat diagram and also explain cut off wavelength, quantum efficiency and  responsivity of a PIN photodetector.