Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: COMPUTER COMMUNICATION						       	       Time: 3 hours

Subject Code: 09EC234				       				                     	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Bring out the significance of Modems.

2.	Why LANs and MANs are unswitched?

3.	What is known as piggybacking at link layer?

4.	A sliding window protocol with a maximum size of 1 for a window uses which steps?

5.	How can you connect two LANs by a router? Describe using a figure.

6.	Give the function of IGMP protocol.

7.	TCP has a special service known as “URGENT Data”. What is that feature?

8.	When does the “silly window syndrome” occur in the Transport layer?

9.	What is meant by a cyber crime?

10.	What types of solutions does DNS provide?

PART – B (5 x 3 = 15 MARKS)

11.	Distinguish TCP from OSI model.

12. 	What is Manchester coding used for in the LAN 802.5 token ring?

13.	Enlist the techniques for achieving good quality of service in ATM networks.

14.	In a transport layer, reason out why the maximum packet life time “T” has to be large enough to ensure that not only the packet but also its acknowledgements have vanished.

15.	Give the issues associated with SMTP protocol.

PART – C (5 x 15 = 75 MARKS)

16.	A host is connected to 16 asynchronous terminals through a pair of statistical time division multiplexers utilizing the multiple character protocol.  The terminal ports operate at 1200 bps.  The line port has a bit rate of 9600 bps.  The data link control protocol is HDLC and the maximum size of the frame is 261 bytes.

a.	Calculate the line utilization efficiency when all ports generate their maximum traffic. Will 	queues be developed for this load ?												(5)

b.	What is the maximum line utilization efficiency without having the queues?			(5)

c.	If the host sends full screen display of average 1200 characters to each terminal will there be any queue?																(5)

(OR)

17.	Summarize the main functions of the various layers of the OSI model.

18.	a.	Check whether the following bytes have any error. The last two bytes are the transport 	protocol checksum bytes.

10100101  00000110   11100010  01110101  11011111  00011111					(8)

b.	Generate a CRC code for the data word 110101010 using the divisor 1010.1			(7)

(OR)

19.	Compare the features of WiFi, IEEE 802.4 and IEEE 802.5 standards.

[P.TO.]

20.	With the layered architecture of packet switched and data switched networks explain how they work.

(OR)

21.	Discuss how the:

a.	Distance Vector Routing 		b.	Link state routing

find their IP address respectively with suitable examples.

22.	a.	Show how a three way hand shake is deployed to establish connection using Transport 			protocols.																	(8)

b.	What are the services provided by the Transport layer to the upper layers?			(7)

(OR)

23.	Analyze how TCP congestion control is maintained.

24.	Write detailed notes on:

i.	electronic mail.																(7)

ii.	world wide web.																(8)

(OR)

25.	Bring out the principles of cryptography and show how it is an essential tool to provide secure communication.