Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title:	ELECTRONICS AND MICROPROCESSORS			       Time: 3 hours

Subject Code:	09EC235			       						              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	A zener diode is used as a  __________ .

2.	A FET is a ___________ controlled device.

3.	List the applications of photo diode.

4.	Draw the equivalent circuit of piezo electric crystal.

5.	Write the truth table of a half subractor.

6.	List the different types of flip flops.

7.	Which register keeps the address of next instruction?

8.	Name two instructions that use immediate addressing mode.

9.	What is EPROM?

10.	What is an interrupt?

PART – B (5 x 3 = 15 MARKS)

11.	Compare the different types of BJT configurations.

12.	State the advantages and disadvantages of thermistor.

13.	With neat diagram explain full adder.

14.	Classify 8085 instruction according to the word size.

15.	Explain DMA data transfer technique.

PART – C (5 x 15 = 75 MARKS)

16.	Explain the operation of centre tapped full wave rectifier with neat diagrams and waveforms.

(OR)

17.	 Describe the operation of class B push pull amplifier and mention its advantages and 	disadvantages.

18. 	Explain the construction of potentiometer and semiconductor strain gauge and list the 	advantages and disadvantages.

(OR)

19.	With neat diagram explain the working principle of digital voltmeter and standard signal 	generator.

20.	List the different types of shift registers and explain the operation of 4 bit serial-in-serial-out 	shift register with the help of diagrams.

(OR)

21.	a. 	Convert (475.25)8, (9B2.1A)H, (1101.1)2 to its decimal equivalent.					(9)

b. 	Convert decimal number 35.45 to octal, hexadecimal and binary. 					(6)

22.	Explain 8085 instructions based on function with examples.

(OR)

23.	Write programs to add and subtract two 8 bit numbers.

[P.T.O]

24.	a.	Explain any temperature control system using 8085 with suitable diagram. 			(8)

b.	compare memory mapped I/O and I/O mapped I/O. 								(7)

(OR)

25. 	What are the different types of data transfer schemes and explain synchronous, asynchronous and interrupt driven data transfer schemes.