Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: BASIC ELECTRICAL ENGINEERING					       Time: 3 hours

Subject Code: 09EE101			       					              	      	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define idle source and dependent source.

2.	State Ohm’s law.

3.	Define self and mutual inductance.

4.	Write the equation for mutual inductance.

5.	What is the significance of RMS value?

6.	Define Power factor.

7.	Why is a single phase induction motor not self starting?

8.	What is shunt – shunt and long shunt?

9.	Why is Earthing needed?

10.	What is the key difference between MC and MI?

PART – B (5 x 3 = 15 MARKS)

11.	Explain current division rule with an example.

12. 	Define the DOT role.

13.	What is a phasor diagram?

14.	Comment why an induction motor is called as a rotating transformer.

15. 	What are the different types of wiring?

PART – C (5 x 15 = 75 MARKS)

16. 	Find the equivalent circuit across the terminal A and B by reducing the given circuit using source transformation.

(OR)

[P.T.O]

17. 	Find the current through the 15 Ω resistor using star delta transformation.

18.	a.	A 400 V, 3φ balanced supply is connected to 8 + j6 ohm balanced star – load with the 			phase sequence RYB. Determine the values of currents and the power consumed by load.

b.	Find the total inductance of the series coils.

(OR)

19. 	Find the average power supplied to the 24 Ω resistor.

20. 	Explain in detail about nuclear power generation with a line diagram.

(OR)

21. 	a.	A sine wave of v(t) = 200sin50t is applied to a 10 Ω resistor in series with a coil. The 			reading of voltmeter is 120 V and  75 V across R and L respectively. Calculate the power 		and 	reactive volt ampere in the coil and power factor of the circuit.

b.	A sinusoidal voltage V = 50 sinωt is applied to a series RL Circuit.  The current in the 			circuit is given by t = 25 sin(ωt-53º).  Determine 		i.	Apparent power	ii.	Power 			factor	iii.	Average Power 	iv.	Instantaneous Power.

22.	a.	With a neat diagram explain the construction and working of a three phase induction 				motor.

b.	Derive the EMF equation of a single phase transformer.

(OR)

23. 	a.	Explain how a single phase induction motor is made self starting.

b.	Derive the EMF equation for a DC generator.

24.	a.	Explain the construction and operating principle of moving iron instruments with an 				example.

b.	Discuss the working of an energy meter with its equation and diagram.

(OR)

25. 	a.	Draw a neat diagram of a staircase wiring and explain its working.

b.	What is the principle of Earthing? Explain the procedure of the same.