Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ELECTRONIC CIRCUITS								      	       Time: 3 hours

Subject Code: 09EE201				       		        		       	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define Regulation.

2.	What is a filter?

3.	Draw the diagram for a high pass filter.

4.	Clampers are also known as_________.

5.	What are the advantages of cascaded BJT amplifier?

6.	The gain of the Darlington amplifier is ___________.

7.	The efficiency of a class C amplifier is____________.

8.	What is the gain of a negative feedback circuit?

9.	The piezo electric material used in a crystal oscillator is__________.

10.	State any one application of a bistable multivibrator.

PART – B  (5 x 3 = 15 MARKS)

11. 	Define transformer utilization factor.

12.	Define Attenuation. Draw the equivalent circuit of an attenuator.

13. 	Write a technical note on the Chopper amplifier.

14. 	In a class A amplifier, VCE(max) = 15 V, VCE(min) = 1 V. Find the overall efficiency for:

a.	Series – fed load		 b.	Transformer coupled load.

15. 	Give the Barkhausen criterion for oscillation.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain the half wave rectifier and full wave rectifier and derive all the parameters.

(OR)

17. 	With a neat block diagram explain the switched mode power supply.

18. 	Obtain the response of a low pass RC circuit for step, pulse and square wave inputs.

(OR)

19. 	a.	Explain the pulse transformer with a neat diagram.								(12)

b.	State the application of a pulse transformer.										(3)

20. 	Give the DC analysis of a differential amplifier using BJT.

(OR)

21. 	Describe in detail about the two stage RC coupled amplifier.

22. 	Explain the current series feedback circuit and derive the various parameters.

(OR)

23. 	Explain in detail about the class B push-pull amplifier with a neat diagram.

24. 	Compare the Hartley and Colpitt’s oscillators. Derive the frequency of oscillation of the 	Colpitt’s oscillator.

(OR)

25. 	With a neat diagram explain the operation of a Schmitt trigger.