Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DIGITAL ELECTRONICS								      	       Time: 3 hours

Subject Code: 09EE202				       			       		       	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Convert the following number:

(247.36)8 = (    )16

2.	Convert the following binary number to an equivalent gray code:

110100.

3.	Show that a full adder can be constructed with two half adders and an OR gate.

4.	Draw the logic diagram for a 2 to 4 line decoder with an enable input.

5.	What is a D Flip Flop?

6.	What is an unstable state?

7.	What is a Hazard?

8.	What is a Race condition?

9.	Define saturated and unsaturated logic. Give examples.

10.	What is propagation delay of a gate?

PART – B  (5 x 3 = 15 MARKS)

11.	Prove the following identity using Boolean algebra and De-Morgan’s theorem.

12.	Draw the logic diagram of a Look ahead carry generator.

13.	What are the types of Multivibrators? Give a definition for each.

14.	Draw the logic circuit for an SR latch using NAND gates.

15.	Compare the performance of TTL and ECL logic families.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	A seven bit Hamming code is received as 1101101. What is the correct code?		(5)	                                                                                                             	b. 	Minimize the 4 variable logic function using tabular minimization:

(5)

c. 	Reduce the following Boolean functions by determining essential prime implicants:

(5)

(OR)

17.	a. 	Minimize  using Karnaugh map. 	        			(5)

b. 	Prove the following Boolean identity: AB +AB+BC=AB+AB+AC.				(5)

c. 	Realize the following logic operations using AOI and NOR implementations.

(5)

18.	With necessary diagrams explain about the 4-bit comparator.

(OR)

19.	Implement the following function with a multiplexer:

[P.T.O]

20.	Explain the operation of a Master- Slave JK FlipFlop.

(OR)

21.	a. 	Design a Mod-6 synchronous counter.				      							(7)

b. 	Design a synchronous counter to count the following random sequence using a JK Flip-			Flop:

0  1   3   7   6   5					       											(8)

22.	Explain the design procedure of an Asynchronous Sequential circuit.

(OR)

23.	Two levels M and N enter a logic circuit. Design an asynchronous circuit such that if N goes high before M, the output will be the AND of M and N. However if M goes high before N, the output must be 0.

24.	Explain the RTL and DTL circuits with necessary diagrams.

(OR)

25.	With a neat circuit diagram, explain the operation of the following:

a.	CMOS -NAND gate.					        									(5)

b.	CMOS - NOR gate.					        									(5)

c.	CMOS - Inverter.						        									(5)