Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: SIGNALS AND SYSTEMS								      	       Time: 3 hours

Subject Code: 09EI218			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	tδ(t) =

a.	t			b.	0			c.	1	 		d.	u(t)

2.	The relation between a signum function and a unit step function is sgn(t) =

a.	2u(t) – 1		b.	u(t) – 1		c.	2u(t) 		d.	u(t) – u(–t)

3.	Fourier series applies to:

a.	only periodic signals					b.	only aperiodic signals

c.	both periodic and aperiodic signals 		d.	only random signals

4.	The spectrum of a rectangular pulse is:

a.	Gaussian function					b.	sinc function

c.	sinc2 function						d.	rectangular function

5.	The DTFT of a sequence x[n] = [1, 1, 1, 1] is:

a.	e-jω/2 sin(2ω) / sin(ω/2) 				b.	 e-jω sin(2ω) / sin(ω/2)

c.	e-j3ω/2 sin(2ω) / sin(ω/2)				d.	e-j2ω sin(2ω) / sin(ω/2)

6.	The frequency response of DTFT is:

a.	continuous							b.	discrete

c.	there is no frequency response 	 	 	d.	none of the above

7.	The minimum sampling rate for band pass signal is:

a.	2fm			b.	fm			c.	fm/2		d.	4fm

8.	The Nyquist rate for the signal x(t) = 5 sinc(200πt) + 3 sinc2 (700πt) is:

a.	1400 Hz		b.	49000Hz	c.	500 Hz	d.	900 Hz

9.	The only signal whose ROC is the entire z-plane is:

a.	δ[n] 		b.	u[n]			c.	 r[n]			d.	 an

10.	Long division method cannot yield a sequence which is:

a.	causal		b.	non-causal	c.	 two-side		d.	 none of the above

PART – B  (5 x 3 = 15 MARKS)

11.	Find whether the signal

x(t) = 	t –2 ;	–2≤ t ≤0

2– t	;	0 ≤ t ≤ 2

0	;	otherwise

is an energy signal or a power signal. Also find the energy and power of the signal.

12.	Determine the Fourier series representation for the signal x(t) = 1 + sin6t + cos4t and plot its magnitude and phase spectrum.

13.	What is aliasing? How can one overcome aliasing effects?

14.	State and prove the time-scaling property of DTFT.

15.	Using properties of Z-transform, find the Z-transform of the signal x[n] = 2n u [n – 2].

[P.T.O.]

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Consider a continuous time system with the following input – output relationship:           	y(t) = x(t / 2). Determine whether the system is									(10)

i.	memoryless		ii.	time-variant		iii.	linear	iv.	causal		v.	stable

b.	The input x[n] and the impulse response h[n] of a DT LTI system are given by:

x [n] = an u[n], 0<a<1;		h[n] = u[n]										(5)

Find the response y[n].

(OR)

17.	a.	Examine whether the given signal is periodic or not. If periodic determine the fundamental 		period. x(t) = sin(10t + 1) – 2cos(5t – 2).										(5)

b.	Convolute the following two continuous time signals:								(10)

x1(t) = e–2t u(t).

x2(t) = u(t + 2).

18.	a.	Find the Fourier Series for the periodic signal x(t) = t for 0 ≤ t ≤ 1 so that it repeats every 1 	second.																		(8)

b.	Find the convolution of the signals x1(t) = 2e–2t u(t) and x2(t) = u(t) using Fourier 				transform.																	(7)

(OR)

19.	Solve the differential equation using FT:

y′′(t) + 4y′(t) + 5y(t) = 3x′(t) + x(t), for all t

where,  x(t) = (1 + e-t) u(t).

20.	Describe the process of natural and flat top sampling. Brief its effects and also technique to overcome it.

(OR)

21.	Analyze different techniques incorporated for reconstruction of CT signal from its samples.

n

X[n]

……

………

3     2   1  0   1     2    3

22.	a.	Determine the Fourier series coefficients for the DT periodic sequence shown in the figure 		below.																		(7)

b.	Find the response y[n] of the DT system	:										(8)

y[n] – 3/4 y[n–1] + 1/8 y[n – 2] = x[n] – 1/2 x[n –1]

for an input x[n] = (1/2)n u[n] using DTFT.

(OR)

23.	a.	State and prove the Parseval’s theorem.											(5)

b.	Using  Parseval’s relation, evaluate the following integral.							(10)

0π45+4cosωdω

24.	a.	Find the Z-transform of x[n] = nαnu[-n].											(7)

b.	Find the inverse Z-transform of: 												(8)

X(Z) = log (1 – ½ Z-1) ; ROC |Z| > ½

(OR)

25.	Solve the difference equation for y[n] using z-transform and the specified initial conditions.

y[n] = y[n –1] – y[n –2] +2 , n ≥ 0

y(–2) =1, y(–1) = 2 .