Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: CONTROL SYSTEMS								            	       Time: 3 hours

Subject Code: 09EI219						       			  		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is electrical analogue of a gear?

2. 	Mention the basic elements of a closed loop system.

3. 	List the standard test signals used in analysis of control systems.

4. 	What is a steady state error?

5. 	What are the frequency domain specifications?

6. 	Define phase cross over frequency.

7. 	What is the condition for the system G(S) = K (S + a) / S(S + b) to have a circle in its root     	locus?

8. 	Define asymptotic stability.

9. 	What are synchros?

10. 	In which situation is fuzzy logic most suitable?

PART – B  (5 x 3 = 15 MARKS)

11. 	What is the mathematical model of a system?

12. 	What is the difference between type and order of a system?

13. 	List any two advantages of frequency response analysis.

14. 	By Routh–Hurwitz criterion, check the condition for stability of the given system:

a0 s4+ a1 s3+ a2 s2+ a3 s+ a4 = 0.

15. 	Draw the Fuzzy Logic control block diagram.

PART – C  (5 x 15 = 75 MARKS)

16. 	Write the differential equations governing the behaviour of the mechanical system shown in the figure below. Draw the force voltage and force current electrical analogous circuits and verify by writing mesh and node equations.

(OR)

[P.T.O]

17. 	Determine the transfer function C(s) / R(s) for the block diagram shown in the figure by drawing Signal Flow Graph and then using Mason’s gain formula.

18. 	The open loop transfer functions of the three systems are given as

a.	4(s+1)(s+2)	b.	2s(s+4)(s+6)	c.	5s2(s+3)(s+10)

Determine the positional, velocity and acceleration error constants for these systems.  Also for the system given in ‘b’ determine the steady state errors with step input r(t)= u(t), ramp input r(t) = t and acceleration input r(t) = (½) t2

(OR)

19.	Obtain the unit – step response of a unity feedback control system whose open –loop transfer function is G(s) = 1/ s(s+1). Obtain also the rise time, peak time, maximum overshoot and settling time.

20. 	Consider a unity feedback control system with the following open-loop transfer function: G(s)=Ks(s2+s+4). Determine the value of the gain K using Bode plot such that the phase margin is 50º. What is the gain margin for this case?

(OR)

21. 	Sketch the polar plot for the given unity feedback system having an open loop transfer functionG(s)=Ks(1+0.5s)(1+5s) and determine the value of   K so that the gain margin is 20 db.

22.	a.	Consider the characteristic equation S4+2S3+(4+K)S2+9S+25=0. Using the 	Routh’s  stability criterion, determine the range of K for stability.

b.	Distinguish between relative stability and absolute stability.

(OR)

23.	State the rules for the construction of Root Locus. State the condition when P, PI, PD and PID

controllers are preferred.

24.	Explain the working of a Tachogenerator with a neat sketch.  List its applications.

(OR)

25.	Discuss about the level controller using Fuzzy network controller.