Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ELECTRICAL MACHINES								       Time: 3 hours

Subject Code: 09EI220				       					              	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The purpose of a commutator is  __________.

2.	Draw the speed torque characteristics of DC series motor.

3.	The advantage of sandwich type of winding in a transformer is ___________.

4.	The transformer short circuit test should be conducted on __________ (low / high) voltage 	side.

5.	Slip ring induction motors have _____________ (higher/ lower) starting torque than squirrel cage induction motors.

6.	At what excitation condition the synchronous motor operates as synchronous condenser?

7.	Single phase induction motors are __________ self starting motors.

8.	____________ (Linear Induction / Universal) motor can operate in both AC and DC supply.

9.	What is the method of heating can be used for a non conducting material?

10.	Give two properties of an ideal traction system.

PART – B (5 x 3 = 15 MARKS)

11.	Justify:  Iron / core losses are constant and will not vary with load change.

12.	What are the applications of transformer equivalent circuits?

13.	Why are the 3 phase induction motors widely used in many applications?

14.	Write any three applications of stepper motors.

15.	What are the important characteristics of welding generators?

PART – C (5 x 15 = 75 MARKS)

16.	a.	Derive the torque equation of the DC motor. 									(8)

b.	Draw the electrical characteristics of a compound generator and explain the compound generator operation in terms of windings connected (short and long) and resultant flux (differential and cumulative). 													(7)

(OR)

17.	a.	What is the necessity of starters in motors? Draw the 3 point starter circuit and explain the 	operation of two protection coils in it. 											(8)

b.	Explain the power electronics based armature voltage control in DC motors. 		(7)

18.	a. 	Derive the EMF equation of the transformer. 									(7)

b.	A 25-kVA transformer has 500 turns on the primary and 50 turns on the secondary winding.  The primary is connected to 3000-V, 50-Hz supply.  Find the full-load primary and secondary currents, the secondary e.m.f. and the maximum flux in the core.  Neglect leakage drops and no-load primary current. 										(8)

(OR)

19.	The O.C and S.C test data are given below for a single phase, 5 kVA, 200V/400V, 50Hz transformer. O.C test from LV side: 200V, 1.25A, 150 W. S.C test from HV side: 20V,  12.5A,  175W. Draw the equivalent circuit of the transformer:	a.	 referred to LV side

b. 	referred to HV side inserting all the parameter values.

[P.T.O]

20.	a.	Explain the different regions of operations in torque slip characteristics of a 3 phase 	induction motor.  																(7)

b.	Draw the star delta starter circuit and briefly explain the operation of it. 			(8)

(OR)

21.	Explain the operation of synchronous motor at constant load with different field excitation. Draw the phasor diagram (normal, under and over excitation), V and inverted V curves of the same.

22.	Explain the operation of single phase shaded pole induction motor with suitable diagrams.

(OR)

23.	Describe the principles and operation of a universal motor with suitable diagrams.

24.	What are the basic principles of induction heating? Explain the ARC furnace and its types.

(OR)

25.	Discuss the recent trends in electrical traction.