Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: LOGIC AND DISTRIBUTED CONTROL SYSTEMS	              Time: 3 hours

Subject Code: 09EI224				       				        		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Draw an example response of a linear system.

2.	Mention the equation of PI Controller.

3.	What does the term ISO slot in PLC refer to?

4.	List out the basic blocks of PLC.

5.	List any two arithmetic functions in PLC programming.

6.	What is an Interlock function in PLC?

7.	Write the syntax of MCR and expand it.

8.	What is meant by redundancy in DCS?

9.	Why is system integration between DCS and computers required?

10.	Write any two case studies of DCS.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the block diagram of a DAS. Give the full form of DAS.

12.	Draw the Ladder Program for automating a Car Park Area and display a light for “House 	Full” 	if the parking has 50 cars. Assume you could count both incoming and 	outgoing cars.

13.	Describe the operation of an on/off controller and explain how it might be used to control the temperature in a domestic central heating system using PLC.

14.	Explain the function of an Operator Station Unit of a DCS.

15.	Explain the concept of HART.

PART – C  (5 x 15 = 75 MARKS)

16.	Along with a clear block diagram compare the direct digital control and supervisory digital control.

(OR)

17.	a. 	How do you realize discrete composite controllers? Focus on Integral and Derivative part.

b. 	What is the significance of Position and Velocity of Algorithms?

18.	Develop a ladder program for the sequential control using Timers. A Lube-oil pump motor starter coil M1 is energized when the start pushbutton PB2 is momentarily actuated. As a result M1-1 control contact closes to seal in M1 and the lube-oil pump motor starts. When the lube-oil pump builds up sufficient oil pressure the lube-oil pressure switch PSl closes. This in turn energizes coil M2 to start the main drive motor and energizes coil lTO to begin the time-delay period. After the preset time-delay period of 15 s 1 TO-l contact closes to energize coil M3 and start the feed motor.

(OR)

19.	Draw the Internal Architecture of the PLC and explain the following:					(5)

a.  	CPU.																		(5)

b.  	I/O Units.																	(5)

[P.T.O]

20.	Explain the following topics:

a. 	PID Function in PLC.															(7.5)

b. 	Skip and MCR Function.														(7.5)

(OR)

21.	List out the sequences of Washing Machine Automation. To implement the control logic using PLC, develop a ladder logic programming. Give a clear description of sensors and output devices.

22.	Draw the building blocks of a Distributed Control System and explain the functionality of Operator Station and Field Control Unit.

(OR)

23.	a. 	What do you mean by Field Control units?

b. 	Explain the term redundancy and its importance in the field of DCS.

24.	Analyze the case study of implementation of DCS in a Steel plant. Point the control objectives clearly.

(OR)

25.	List out the supervisory computer tasks that could be scheduled in DCS and its configuration.