Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: UNIT OPERATIONS IN FOOD PROCESSING II		      	       Time: 3 hours

Subject Code: 09FP206			       						            	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define the term sedimentation.

2.	Name the three forces acting on a particle in a centrifugal field.

3.	Define compressibility of the filter cake.

4.	What is meant by constant rate filtration?

5.	Define osmotic pressure.

6.	What is membrane fouling?

7.	What are high shear mixers and write their applications in food industries.

8.	Define mixing time.

9.	Write a general equation giving the power required for size reduction.

10.	Give any two examples of coarse and fine crushing equipment.

PART – B (5 x 3 = 15 MARKS)

11.	Write the principle involved in a centrifugal separator.

12.	What are filter aids and write about their significance.

13.	Using the Van't Hoff equation calculate the osmotic pressure of a 0.9% NaCl solution at 370C.

14.	Draw a flow pattern produced by a radial-flow impeller in a baffled tank.

15.	Write Bond’s law and explain work index.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Derive an equation of radial variation in pressure across the centrifuge in liquid - liquid 	separation.																	(7)

b.	Explain the principle and working of the separation of cream from milk in the centrifugal 			separator.																	(8)

(OR)

17.	a.	Write a note on sedimentation of particles in a gas.								(7)

b.	Calculate the settling velocity of dust particles of 	a.	60 μm 	b.	10 μm diameter in air at 21°C and 100 kPa pressure. Assume that the particles are spherical and of density 1280 kg m-3, and that the viscosity of air =1.8 x 10-5 N s m-2and density of air = 1.2kg m3. Assume Stokes' Law.															(8)

18 	A filtration test was carried out, with particular product slurry, on a laboratory filter press 	under a constant pressure of 340 kPa and volumes of filtrate were collected as follows:

| Filtrate volume (kg) | 20 | 40 | 60 | 80 |
| --- | --- | --- | --- | --- |
| Time (min) | 8 | 26 | 54.5 | 93 |

The area of the laboratory filter was 0.186 m2. In a plant scale filter, it is desired to filter a slurry containing the same material, but at 50% greater concentration than that used for the test, and under a pressure of 270 kPa. Estimate the quantity of filtrate that would pass through in 1 hour if the area of the filter is 9.3 m2.

(OR)

[P.T.O]

19.	With a neat diagram explain the principle and working of plate and frame filter press and 	rotary 	filter.

20.	A solution of sucrose in water at 25°C is to be concentrated by reverse osmosis. It is 	found that, with a differential applied pressure of 5000 kPa, the rate of movement of the water molecules through the membrane is 25 kg m-2 h-1 for a 10% solution of sucrose. Estimate the flow rate through the membrane for a differential pressure of 10,000 kPa with the 10% sucrose solution, and also estimate the flow rate for a differential pressure of 10,000 kPa but with a sucrose concentration of 20%. For sucrose, the molecular weight is 342 so for a 10% solution, molar concentration is 0.304 moles m-3 and for 20%, 0.632 moles m-3. The gas constant is 8.314 kPa m.

(OR)

21.	Explain in detail of reverse osmosis processes, pretreatment process, membrane 	assembly and its various applications.

22.	In a batch mixer, blending starch and dried-powdered vegetables for a soup mixture, the 	initial proportions of dried vegetable to starch were 40:60. If the variance of the sample compositions measured in terms of fractional compositions of starch was found to be 	0.0823 after 300 s of mixing, for how much longer should the mixing continue to reach 	the specified maximum sample composition variance for a 24 particle sample of 0.02? Assume that the starch and the vegetable particles are of approximately the same physical size.

(OR)

23.	Write a note on the following 		a.	 powder and particle mixers 	b. dough and paste 	mixers.																			(7+8)

24. 	a.	Derive a material balance over a screen and derive an equation for screen effectiveness. (7)

b.	Explain the principle and working of a Hammer mill. 								(8)

(OR)

25.	a.	Explain Kick’s law and Rittinger’s law of crushing.								(6)

b.	It is found that the energy required to reduce particles from a mean diameter of 1 cm to 	   0.3 cm is 11 kJ  kg-1. Estimate the energy requirement to reduce the same particles from a diameter of 0.1 cm to 0.01 cm assuming:

i.	Kick's Law,

ii.	Rittinger's Law,

iii.	Bond's Equation.															(9)