Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DIGITAL PRINCIPLES AND SYSTEM DESIGN			       Time: 3 hours

Subject Code: 09IT204			       						                     Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Given that 1610 = 100b, find the value of b.

2.	Is Gray code a BCD code?

3.	What is a comparator?

4.	Is Verilog HDL a case sensitive language?

5.	Why is a multiplexer called a data selector?

6.	Is the PROM volatile or non-volatile?

7.	How many flip-flops are required for storing n bits of information?

8.	What do you mean by presetting the counter?

9.	Static hazards are of how many types? Name them.

10.	Asynchronous sequential circuit is easier to design. Say True or False.

PART – B (5 x 3 = 15 MARKS)

11.	Prove that ab+a'c=(a+c)(a'+b).

12.	Write the HDL code for full adder circuit.

13.	Use a 4x1 MUX to implement the logic function:

F(A,B,C) = ∑m(1,2,4,7)

14.	Write the excitation table for a JK flip-flop.

15.	Draw the block diagram of the Asynchronous sequential Circuit.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Each of the following numbers is a signed binary number. Determine the decimal value in 	each case, if they are in 	a.		sign-magnitude form		b.	 2’s complement form 	c.	 1’s 	complement form.						   							(12)

i.	01101		ii.	 010111		iii.	10111 	iv.	1101010

b.	Multiply 1011.1012 by 101.012.												  (3)

(OR)

17.	a.	Derive a logic expression that equals 1 only when the two binary numbers A1A0 and B1B0 	have the same value. Draw the logic diagram and construct the truth table to verify the 	logic.																		(10)

b.	State and explain the principle of Duality.						    				(5)

18.	a.	Design the full subtractor circuit and write the HDL code using dataflow description.																						(10)

b.	Design a combinational circuit with three inputs and one output. The output is one when

the binary value of the input is less than 3. The output is 0 otherwise.                 		(5)

(OR)

19.	Design a 4-bit Binary to Gray code converter and write the HDL code for the same.

20.	a.	Write the HDL code for a two-to-four-line decoder using dataflow description.      	(5)

b.	Implement the following Boolean function with 4x1 multiplexer and external gates.

F(A,B,C,D)=∑(1,3,4,11,12,13,14,15)						 					(10)

[P.T.O]

(OR)

21.	a.	Specify the truth table of an Octal-to-binary priority encoder. Provide an output V to 	indicate that at least one of the inputs is present. The input with the highest subscript  	number has the highest priority. What will be the value of the four outputs if inputs D2 and 	D6 are 1 at the same time?														 (10)

b.	Write a Verilog model for a 4:2 encoder circuit.					 				 (5)

22.	A sequential circuits with two D flip-flops A and B, two inputs x and y, and one output z is

specified by following next-state and output equations.

A(t+1)=x'y+xBB(t+1)=x'A+xBz=A

a.	Draw the logic diagram of the circuits

b.	List the state table for the sequential circuit.

c.	Draw the corresponding state diagram

(OR)

23.	Write and verify a Verilog model of a D flip-flop having synchronous reset.

24.	Draw a PLA circuit to implement the functions:

F1=A'B+AC'+A'BC'F2=(AC+AB+BC {)'

(OR)

25.	Derive the circuit for the following flow table:

Fig: Two states with two inputs and one output