Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MICROPROCESSORS AND INTERFACING		      		       Time: 3 hours

Subject Code: 09IT206				       				        		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	How large is 8086’s I/O address space?

2. 	What two elements are combined to form a physical address?

3. 	Which of the 8086’s internal registers are used for memory segmentation?

4. 	Which instruction of 8086 performs the following operations?

a.	EA(Reg16), EA+2(DS)

b.	0-(D)(D)

5. 	Name one signal that is supplied by 8088 but not by 8086.

6. 	At what speeds are 8088s generally available?

7. 	If ports A, B and C of an 82C55 A are to be configured for mode 0 operation where the A and

B ports are inputs and C is an output port, what is the control word?

8. 	List three core input/output interfaces commonly used in microcomputer systems.

9. 	What name is given to the special software routine to which control is passed when an interrupt 	occurs?

10. 	What does STI stand for?

PART – B  (5 x 3 = 15 MARKS)

11. 	If the current values in the stack segment register and stack pointer are C00016 and FF0016 	respectively, what is the address of the current top of the stack?

12. 	If the value if AL is initially FFh  what will be the value in AL after executing the following       	instructions?

MOV CL, 04

SHL AL, CL

13. 	Describe the function of Queue status signals.

14. 	Specify the mode of operation for the keyboard and display when an 8279 is configured with       	command word 0 equal to 3F.

15. 	Explain the types of interrupts with their priorities.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain the internal architecture and the software model of Intel 8086 processor in detail.

(OR)

17.  a. 	What is the data word shown in the figure below? Express the result in hexadecimal form. 		Is it stored at an even- or odd-addressed word boundary? Is it an aligned or misaligned            		word of data? 																(5)                                                                                                	    																																				[P.T.O]

b.	Explain the memory segmentation and segment registers in 8086 Microprocessor.	(10)

18. 	Discuss the different addressing modes supported in 8086 processor.

(OR)

19. 	a. 	What are the arithmetic and string instructions available for computation in 8086?  Explain 	in  detail.                                                                                                    				(10)

b. 	Differentiate logical shift and arithmetic shift with a suitable example.         	        	(5)

20.	a. 	How is the 8086 microprocessor configured in minimum mode and maximum mode?

(2)

b.	Explain the minimum mode and maximum mode interface signals in detail.        		(13)

(OR)

21. 	Describe the hardware organization of memory in 8086 and 8088 with the illustration of word and byte transfer in both 8086 and 8088.

22. 	a. 	Draw the command word and explain the control bit functions of 8255 programmable 	peripheral interface.															(5)

b. 	Describe the different operating modes of 82C54 counters.	                              	(10)

(OR)

23. 	Draw the block diagram of DMA and explain how it performs the data transfer in between I/O

and memory unit.

24. 	a. 	What are Non maskable interrupts?			        	                                 			(3)

b. 	Explain the internal architecture of programmable interrupt controller.				(12)

(OR)

25.  	a. 	How is the Interrupt controller initialized? Explain it using a flow chart.              		(5)

b. 	Discuss the Initialization command words and operational command words of the         			Interrupt controller.                                                                                                  		(10)