Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ENGINEERING MECHANICS						             	       Time: 3 hours

Subject Code: 09ME201				       				              	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	State the SI unit of Energy.

2.	What is meant by the Principle of Transmissibility?

3.	Sketch the freebody diagram of a rigid body.

4.	State Varignon’s theorem.

5.	Give the significance of the Polar Moment of Inertia.

6.	Define radius of Gyration.

7.	State Newton’s second law of motion.

8.	Write the Work Energy equation.

9.	Define belt friction.

10.	What is the co-efficient of static friction of leather on metal?

PART – B  (5 x 3 = 15 MARKS)

11.	State the laws of mechanics.

12. 	List the types of couples and give the difference between moment and couple.

13.	Derive the Moment of Inertia for a rectangular section.

14.	Write the work energy equation of a particle.

15. 	Write the Coulomb’s laws of dry friction. Differentiate between static friction and dynamic friction.

PART – C  (5 x 15 = 75 MARKS)

16. 	The lines of action of three forces are concurrent at the origin ‘O’, pass through points A, B and C having coordinates, (3, 0, 3), (2, 2, 4), and (1, 2, 4) respectively. If the magnitude of the forces is 10N, 30N, and 40N, find the magnitude and their resultant.

(OR)

17. 	a.	Elucidate the system of forces with neat sketches.								(8)

b.	Define Principle of Equilibrium. Give its features and application.					(7)

18.	a.	Both A and B of weight 200 N and 100 N respectively, rest on a 30o inclined plane and are

attached to the post which is held perpendicular to the plane by force P, parallel to the plane. Assume that all surfaces are smooth and that the cords are parallel to the plane. Determine the value of P. Also find the normal reaction of blocks A and B.			(8)

P

B

A

0

0.5 m

0.25 m

0.25 m

30o

[P.T.O]

L

P

C



a

B

A

b.	A slender rod of length L is lodged between peg C and the vertical wall. It supports a load P at end A. Neglecting friction and the weight of the rod, determine the angle  corresponding to equilibrium.													(7)

(OR)

19. 	A 300x150 mm plate is subjected to four forces. Find the results of the four forces and the two

points at which the line of action of the resultant intersects the edge of the plate.

200 N

A

B 250 N

F

150 mm

150 mm

E

D

C

100 N

250 N

150mm

20.	a.	What are the steps followed to find the centroid of a rectangle?						(8)

b.	Derive the centroid of a Right angled triangle.									(7)

(OR)

21. 	 Find the centroid and moment of inertia of the following section.

10 mm

400 mm

10 mm

10 mm

30 mm

22.	Two weights 80 N and 20 N are connected by a thread and move along a rough horizontal  plane under the action of force 40 N, applied to the first weight of 80 N. The coefficient of friction between the sliding surfaces of the weights and the plane is 0.3. Determine the acceleration of the weights and the tension in the thread using work energy equation.

20 N

80 N

40 N

(OR)

23. 	Two adjacent bullets having the same velocity of 360 meters per second fire simultaneously at angles of 1 and 2 respectively for the same target at a range of 4320 meters. Calculate the time difference between the hits.

[P.T.O]

24. 	 Block (2) rests on Block (1) and is attached by a horizontal rope AB to the wall. What force P is necessary to cause motion of block (1) to impend. The coefficient of friction between the blocks is 1/4 and between the floor and the block (1) is 1/3. Mass of blocks (1) and (2) are 14 kg and 9 kg respectively.

(1)

(2)

45o

A

B

P

(OR)

25.	A block overlying a 10o wedge on a horizontal floor and learning against a vertical wall and weighing 1500 N is to be raised by applying a horizontal force to the wedge. Assuming co-efficient of friction between all the surfaces in contact to be 0.3, determine the minimum horizontal force to be applied to raise the block.

1500 N

100