Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ENGINEERING THERMODYNAMICS					       Time: 3 hours

Subject Code: 09ME202				       				                     	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is meant by path function?

2.	Temperature is example for ______ property.

3.	Name the four processes involved in Carnot cycle.

4.	What is meant by available energy?

5.	Define triple point for pure substance.

6.	The pressure of water at critical point is _____ bar.

7.	State Gay Lussac’s law.

8.	What is meant by stochiometric mixture?

9.	For the same compression ratio, the efficiency of dual combustion cycle is ______ than Diesel cycle.

10.	Name the four processes involved in Brayton cycle.

PART – B (5 x 3 = 15 MARKS)

11.	What are closed, open and isolated systems? Give suitable examples.

12.	Explain briefly clausius inequality.

13.	What are wet, dry and superheated steams?

14.	Write Vander Waals equation of state. How does it differ from the ideal gas equation of state?

15.	In a diesel engine, the compression ratio is 13 and the cut-off ratio is 1.96.Find the air standard efficiency of the engine. Take γ for air is 1.4.

PART – C (5 x 15 = 75 MARKS)

16.	3 Kg of gas at 75°C is heated at constant volume until its pressure is 2 times the initial pressure.

Determine:  a. 	The Final temperature 	b.    The heat transfer 	    c. Change in internal 	energy    d.	Change in Enthalpy 	e.	Change in Entropy.

Assume Cp=1.00 kJ/kgK and CV=0. 71 kJ/kg K.

(OR)

17.	Air flows steadily at the rate of 15 kg/min through an air compressor, entering at 5 m/s velocity, 1bar pressure, and 0. 5 m3/kg volume and leaving at 7.5 m/s, 7 bar, and 0.15 m3 /kg. The internal energy of the air leaving is 165kJ/kg greater than that of the air entering. Cooling water in the compressor jackets absorbs heat from the air at the rate of 125kW.

a.	Compute the rate of shaft work input to the air in kW

b.	Find the ratio of the inlet pipe diameter to outlet pipe diameter.

18.	Two Carnot engines A and B are operated in series. The first engine A receives heat at 870K 	and rejects heat to a reservoir at T2.The second engine B receives the heat rejected by the first 	engine and rejects heat to a reservoir at 300 K. Calculate the intermediate temperature T2 in 0C 	for the following cases.

a.	The work output of the two engines are equal and

b.	The efficiencies of the two engines are equal.

[P.T.O]

(OR)

19.	a.	one kg of water at 273 K is brought into contact with a heat reservoir at 373 K. When the 			water has reached 373 K, find the entropy change of the water, the reservoir and the 				universe.

b.	If water is heated from 273 K to 373 K by first bringing it in contact with a reservoir at 			323K and then with a reservoir at 373 K , what will be the entropy change of the universe?

20.	Steam initially at 0.3 MPa, 250° C is cooled at constant volume.

a.	At what temperature will the steam become saturated vapour?

b.	What is the quality at 80° C?

c.	What is the heat transferred per kg of steam in cooling from 250° C to 80° C?

(OR)

21.	a	Draw and explain the phase equilibrium diagram for a pure substance on p-v coordinates. 																					(8)

b.	Draw and explain the phase equilibrium diagram for a pure substance on p-T coordinates.

(7)

22.	With a neat sketch explain the working of Junker’s calorimeter.

(OR)

23.	A vessel of volume 0.4 m3 contains 0.45 kg of carbon monoxide and 1 kg of air, at 150C. Calculate the partial pressure of each constituents and the total pressure in the vessel. The air contains 23.3% oxygen and 76.7% nitrogen by mass. Take the molar masses of carbon monoxide, oxygen and nitrogen as 28, 32 and 28 kg /kmol, respectively.

24.	An engine working on the Otto cycle is supplied with air at 103 KPa and 27C. The compression ratio is 8. Heat supplied is 1850 kJ/kg. Calculate the maximum pressure and temperature of the cycle, the cycle efficiency and the mean effective pressure. For air take Cp=1.005 kJ/kg K, Cv=0.718 kJ/kg K and R = 0.287 kJ/kg K.

(OR)

25.	Explain the working of a Rankine cycle with T-s and h-s diagram. Also derive the expression for its efficiency.