Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: THERMAL ENGINEERING								       Time: 3 hours

Subject Code: 09ME203			       				             			       Maximum Marks: 100

#### Answer ALL questions

Use of standard thermodynamic tables, Mollier diagram, Psychrometric chart and Refrigerant, property tables are permitted.

PART – A (10 x 1 = 10 MARKS)

1. 	A Steam engine is an example of a __________ engine.

2.	Engine friction is usually expressed in terms of __________ power.

3.	If the value of γ = 1.4, the critical pressure ratio of the nozzle is __________.

4.	De-Lavel turbine is an example of __________.

5.	The pressure during suction stroke is __________ atmospheric pressure.

6.	Expand the term FAD	.

7.	The heat transfer equivalent of one ton of refrigeration is __________ J/s.

8.	__________ is the refrigerant used in aqua ammonia absorption system.

9.	In split air-conditioning system, only __________ is inside the room.

10.	If the air has maximum quantity of water vapour in it then it is said to be __________.

PART – B (5 x 3 = 15 MARKS)

11.	Define knocking in a spark ignition engine.

12.	Sketch the TS diagram of a reheat cycle and mention the different processes.

13.	What is the main advantage of a rotary compressor? List down its types.

14.	The coefficient of performance of a refrigeration system is 7. If the capacity is one ton, calculate the required input energy.

15.	When air is cooled from 40°C DBT to 20°C DBT, what will happen to its specific humidity, relative humidity and enthalpy?

PART – C (5 x 15 = 75 MARKS)

16.	a.	Compare SI and CI engine with respect to basic cycle, ignition, compression ratio, weight 	and thermal efficiency. 														(5)

b.	Explain with a neat sketch the working principle of a fuel injector.					(10)

(OR)

17.	a.	With a neat sketch briefly explain the battery ignition system.						(10)

b.	Find the brake specific fuel consumption in kg/kW of a diesel engine whose fuel consumption is 5 grams per second when the power output is 80 kW. If the mechanical efficiency is 75%, calculate the indicated specific fuel consumption.				(5)

18.	a.	Consider a steam power plant operating on the ideal regenerative Rankine cycle with one 	open feed water heater. Steam enters the turbine at 15 MPa and 600°C and is condensed in 	a condenser at a pressure of 10 kPa. Some steam leaves the turbine with an enthalpy of 	2860.2 kJ/kg and enters the open feed water heater. Determine the fraction of steam 	extracted from the turbine. Neglect pump work. 									(5)

[P.T.O]

b.	The blade tips of a reaction turbine are inclined at angles 30° and 15° in the direction of motion. The guide blades are of the same shape as the moving blades but reversed in direction. The drum diameter is 1.5 m and blades are 15 cm high. The steam has a pressure of 1.86 bar and 0.94 dryness. The speed of the turbine is 3000 rpm and the steam passes through the blades without shock. Draw the velocity triangle.						(5)

c.	Briefly explain the velocity compounding of a turbine, with a neat sketch. 			(5)

(OR)

19.	Derive an expression for the pressure ratio and maximum mass flow rate (any one form) through the nozzle.

20.	A two stage air compressor with perfect inter cooling takes in air at 1 bar and 300 K. The law of compression in both the stages is pv1.3 = constant. The compressed air is delivered at 9 bar. Calculate for unit mass flow rate of air, the minimum work done and the heat rejected to the intercooler. Compare the values if the compression is carried out in a single stage compressor.  Plot the pressure volume diagram.

(OR)

21.	Derive an expression for the shaft work of a reciprocating air compressor assuming zero clearance volume. Also prove that the clearance has no effect on the theoretical work done in compressing and delivering1 kg of air if the suction and delivery pressures, law of compression and expansion processes remain the same.

22. 	A refrigerator uses refrigerant R134a as the working fluid and operates on an ideal vapour compression refrigeration cycle between 0.14 MPa and 0.8 MPa. If the mass flow rate of the refrigerant is 0.05 kg/s, determine the rate of heat removal from the refrigerated space, power input to the compressor, rate of heat rejection to the environment and coefficient of performance of the refrigerator. Sketch the cycle on the PV and TS diagram.

(OR)

23.	With the help of a schematic explain the working principle of an aqua ammonia vapour absorption refrigeration system.

24. 	a.	Find the wet bulb temperature, dew point temperature, specific humidity, specific enthalpy 	and specific volume of humid air whose dry bulb temperature is 29°C and relative 	humidity is 45%. Use psychrometric chart.										(10)

b.	The sensible and latent heat gains in an air-conditioned space are 75 kW and 15 kW respectively. The space is to be maintained at 25°C DBT and 50% RH. The outdoor air is at 40°C DBT and 40% RH. For ventilation purpose outdoor air is mixed up to 25%. Determine the enthalpy of air after mixing and the room sensible heat factor	.		(5)

(OR)

25.	a.	The sling psychrometer reads 40°C DBT and 28°C WBT. By using empirical relations, 	calculate specific humidity, relative humidity, dew point temperature and enthalpy of 	mixture per kg of dry air. Assume pressure to be 1.03 bar.							(10)

b.	The following data were observed during the testing of an air-conditioning system in a 	laboratory.

Air inlet conditions: 	DBT = 35°C and WBT = 24°C;

Air outlet conditions: 	DBT = 24°C and WBT = 21°C;

Velocity of air = 1.5 m/s;

Area of cross section = 0.0625 m2.

Determine the amount of heat removed from air. 									(5)