Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: HEAT AND MASS TRANSFER							       Time: 3 hours

Subject Code: 09ME204						   		        	      	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is the unit for thermal conductivity?

2. 	Mention the formula for critical radius of insulation for a cylinder.

3. 	Define ‘Biot number’.

4. 	What is the restrictive for the application of the Heisler charts?

5. 	Define thermal boundary layer thickness.

6. 	Define forced convection.

7. 	What are the types of heat exchangers?

8. 	What is meant by LMTD?

9. 	Define black body.

10. 	What is meant by monochromatic emissivity?

PART – B  (5 x 3 = 15 MARKS)

11. 	Derive the formula for heat transfer through a single wall.

12.  Briefly discuss about the types of fins.

13. 	What do you understand by the boundary layer concept?

14. 	What are the different modes of mass transfer?

15. 	Explain the types of reflection of radiation.

PART – C  (5 x 15 = 75 MARKS)

16. 	A mild steel tank of wall thickness 10 mm contains water at 90°C whereas atmospheric temperature is 15°C. The thermal conductivity of mild steel is 50 W/m K and heat transfer coefficients for the inside and outside of the tank are 2800 and 11 W/m2 K respectively. Calculate:

a.	Rate of heat loss per unit area of tank surface.

b.	Temperature of the outside surface of the tank.

(OR)

17. 	Derive an expression for critical radius of insulation for	i.	a cylinder 	ii.	a sphere.

18. 	A copper plate 2 mm thick is heated  upto 400° C and then quenched in water at 25° C. Find the Biot number and verify if the lumped heat capacity analysis is applicable. Also find the         a.	time constant		b.	 time required for the plate to reach the temperature of 40° C.

The data given is: h= 100 W/m2K,   Cp= 400 J/Kg K,   k=385 W/m K, plate size = 25 m x 25 m,  ρ = 8800 kg/m3.

(OR)

19. 	How can the transient heat conduction for a plain wall be analyzed with the help of Heisler charts.

20. 	Air flows across a 10 mm diameter sphere at 1 atmosphere pressure and 25°C with a velocity of 5 m/s. An electric coil inside the sphere maintains the surface temperature at 80°C. Calculate the heat lost by the sphere.

[P.T.O]

(OR)

21.  	A vertical plate 0.2 m in height and 1 m wide is maintained at a uniform temperature of 110° C. It is placed in quiet ambient air at 30° C. Find the heat loss from the two surfaces of the plate. Properties of air of 70° C are:  ρ =1.029 Kg/m3, µ = 20.6 x 10-6 Poise, Pr = 0.694 and k =0.0296 W/mK.

22. 	Derive an expression for LMTD of a parallel-flow heat exchanger.

(OR)

23. 	A double pipe counter-flow heat exchanger is used to cool 10,000 kg/h of an oil                     (Cp= 2095 Joules/kg k) from 80 to 50°C using 8000 kg/h water (Cp= 4180 J/kg k) entering at 25°C. Find the heat exchanger area required if the overall heat transfer coefficient is 300 W/m2.

24. 	Explain the various regimes of the saturated pool boiling.

(OR)

25. 	What is the purpose of radiation shield? Find the ratio of heat transfer between two infinite parallel surfaces with radiation shield and without radiation shield.