Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MECHANICS OF MACHINES – I						      	       Time: 3 hours

Subject Code: 09ME205							  		    	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is the difference between a structure and a mechanism in terms of mobility?

2. 	Give an example of an inversion of single slider crank chain.

3. 	Due to slip of the belt, the velocity ratios of the belt drive will ___________.

4. 	Why is a multiple disc clutch used in motorcycles?

5. 	Why is the offset provided to a cam follower mechanism?

6. 	What type of follower motion is used in valve opening and closing of automobile engines?

7. 	What is ‘backlash’ in a gear drive?

8. 	Which gear train is used in fully automatic gearbox of a car?

9. 	Sensitiveness of a governor in terms of its speed is given by___________.

10. 	The right hand side of the ship when viewed from stern is called ___________.

PART – B  (5 x 3 = 15 MARKS)

11. 	State the Kutzbach criterion for the mobility of a planar mechanism with an example.

12. 	What is the phenomenon of ‘slip’ in a belt drive?

13. 	Sketch the following types of followers and state their limitations:

a.	 knife edge follower  	b.	roller follower  	c.	flat faced mushroom follower

14. 	Indicate the following terms with the help of a neat sketch:

a.	 circular pitch  		b.	clearance  		c.	tooth thickness.

15. 	What is the function of a governor? How does it differ from that of a flywheel?

PART – C  (5 x 15 = 75 MARKS)

16. 	Briefly explain any five types of joints used in mechanisms and their degrees of freedom with

neat sketches.

(OR)

17. 	The crank and connecting rod of a crank slider mechanism are 0.5m and 2m long respectively. The crank makes 180 r.p.m in the clockwise direction. When it has turned 45⁰ from the inner dead centre position, determine: 	a. 	Velocity of piston. 		b. Angular velocity of connecting rod 		c. 	Velocity of point E on the connecting rod 1.5m from the slider pin

d.	 Position and linear velocity of any point G on the connecting rod which has the least velocity relative to crank shaft.

18. 	Two pulleys, one 450 mm diameter and the other 200 mm diameter are on parallel shafts 1.95m apart and are connected by a crossed belt. Find the length of the belt required and the angle of contact between the belt and each pulley. What power can be transmitted by the belt when the larger pulley rotates 200 rev/min, if the maximum permissible tension in the belt is 1 kN, and the coefficient of friction between the belt and pulley is 0.25?

(OR)

[P.T.O]

19. 	A single dry plate clutch transmits 7.5 kW at 900 r.p.m. The axial pressure is limited to          0.07 N/mm2. If the coefficient of friction is 0.25, find 	a. 	Mean radius and face width of the friction lining assuming the ratio of the mean radius to the face width as 4	b. 	Outer and inner radii of the clutch plate.

20. 	A cam is to give the following motion to a knife-edged follower:

a.	Outstroke during 60° of cam rotation	b.	Dwell for the next 30° of cam rotation

c. 	Return stroke during next 60° of cam rotation	d. 	Dwell for the remaining 210° of cam rotation.

The stroke of the follower is 42 mm and the minimum radius of the cam is 50 mm. The 	follower moves with a uniform velocity during both outstroke and return strokes. Draw the 	profile of the cam when the axis of the follower passes through the axis of the cam shaft.

(OR)

21.	Design a cam for operating the exhaust valve of an oil engine. It is required to give equal uniform acceleration and retardation during opening and closing of the valve each of which corresponds to 60° of cam rotation. The valve must remain in the fully open position for 30° of cam rotation. The lift of the valve is 36 mm and the least radius of the cam is 40 mm. The follower is provided with a roller of radius 10 mm and its line of stroke passes through the axis of the cam.

22.	Two involute gears of 20° pressure angle are in mesh. The number of teeth on pinion is 20 and the gear ratio 2. If the pitch expressed in module is 5mm and the pitch line speed is 1.2 m/s, assuming addendum as standard and equal to one module, find:

a.	The angle turned through by pinion when one pair of teeth is in mesh.

b.	The maximum velocity of sliding.

(OR)

An epicyclic gear consists of three gears A, B and C as shown in figure. The gear A has 72 internal teeth and gear C has 32 external teeth. The gear B meshes with both A and C and is carried on an arm EF which rotates about the centre of A at 18 r.p.m. If the gear A is fixed, determine the speed of gears B and C.

23.

F

B

E

C

A

24.	The arms of a Porter governor are each 250 mm long and pivoted on the governor axis. The mass of each ball is 5 kg and the mass of the central sleeve is 30 kg. The radius of rotation of the balls is 150 mm when the sleeve begins to rise and reaches a value of 200 mm for maximum speed. Determine the speed range of the governor. If the friction at the sleeve is equivalent of 20 N of load at the sleeve, determine how the speed range is modified.

(OR)

25.	The turbine rotor of a ship has a mass of 3500 kg. It has a radius of gyration of 0.45 m and a speed of 3000 r.p.m. clockwise when looking from stern. Determine the gyroscopic couple and its effect upon the ship:

a.  	when the ship is steering to the left on a curve of 100 m radius at a speed of 36km/h.

b.	when the ship is pitching in a simple harmonic motion, the bow falling with its maximum velocity. The period of pitching is 40 seconds and the total angular displacement between the two extreme positions of pitching is 12 degrees.