Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DESIGN OF MACHINE ELEMENTS					       	       Time: 4 hours

Subject Code: 09ME206				       				              	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define: “Design”.

2.	What is an impact load?

3.	Define Fatigue.

4.	What is the use of S-N curve?

5.	What are types of keys?

6.	Mention the types of Coupling.

7.	A transmission shaft subjected to bending loads must be designed on the basis of ____________.

8.	Write the applications of Cotter joint.

9.	State the function of a Flywheel.

10.	Write down the formula for the maximum torque that can be transmitted by the flywheel of a  hollow shaft.

PART – B (5 x 3 = 15 MARKS)

11. 	List out the mechanical properties of materials.

12. 	Write the Sodenberg and Goodman equation.

13. 	What is a Shaft?

14.	What are the advantages of Knuckle joints?

15. 	What is the Flywheel effect?

PART – C (5 x 15 = 75 MARKS)

16.	A circular bar is simply supported with a span of 0.5 m and is subjected to a concentrated cyclic load at its midspan. The load varies from a minimum value of 20 KN to maximum value of 45 KN.The load direction is transverse to the shaft axis. Decide upon the diameter of the bar taking a factor of safety of 1.5 and factor of 0.85 and 0.89 respectively for size effect and surface finish. Take often following values for material properties. Ultimate strength = 650N/mm2, Yield strength = 450N/mm2, Endurance strength =350N/mm2.

(OR)

17.	A steel member is subjected to a 3-D stress system and resulting principal stress are 120N/mm2 tension, 80N/mm2 and 40N/mm2 compression. If the proportional limit of the material in simple tension is 280N/mm2 and its Poisson’s ratio is 0.3, determine the factor of safety according to 		a.	Maximum principal stress theory 	b.	Maximum principal strain theory 		c.	 Maximum shear stress theory.

18.	The bending stress in a machine part fluctuates between a tensile stress of 280 N/mm2 and a compressive stress of 140 N/mm2.What should be the minimum ultimate tensile strength of this part to carry this fluctuation indefinitely according to:

a.	Goodman’s formula 		b.	Soderberg formula.

Factor of safety is 1.75. Assume that the yield point is never likely to be less than 55% of the ultimate tensile strength or greater than 93% of it.

[P.T.O]

(OR)

19.	A helical valve spring is to be designed for an operating load range of 90 N to 135 N. The deflection of the spring for this load range is 7.5 mm. Assuming a spring index of 10, a permissible shear stress of 480 N/mm2 and a modulus of rigidity of 0.8X105 N/mm2 for the material, determine the dimensions of the spring.

20.	A line shaft rotating at 200 rpm is to transmit 20 KW power. The allowable shear stress for the shaft material is 42 N/mm2. If the shaft carries a central load of 900 N and is simply supported between bearings 3 meters apart, determine the diameter of the shaft. The maximum tensile or compressive stress is not to exceed 56 N/mm2.

(OR)

21.	Design and sketch the protective type C.I flange coupling to transmit 10 kW at 250 rpm.The permissible shear stress for key, shaft, and bolt can be taken as 50 N/mm2.Take crushing stress of key as 90 N/mm2 and shear stress for C.I as 14 N/mm2. Assume maximum torque is 30% higher than mean torque.

22.	Two plates of 10 mm thickness each are to be to be joined by means of a single riveted double strap butt joint. Determine the rivet diameter, rivet pitch, strap thickness and efficiency of the joint. Take the working stress in tension and shearing as 80 MPa and 60 MPa respectively.

(OR)

23.	Design and draw the knuckle joint for a tie rod of circular section to sustain a maximum pull of 70 kN. The ultimate strength of material of the rod against the tearing is 420 MPa. The ultimate tensile and shearing strength of the pin material are 510 MPa and 396 MPa. Determine the tie rod section and the pin section.

24.	Determine the maximum tensile stress in the thin rim of a steel flywheel rotating at 600 rpm. The mean radius of rim is 1500 mm.The fly wheel rim is 200 mm thick and 300 mm wide. The area of cross section of the rim is 0.06 m2 and each of the six spokes is constant in cross section area of 0.01 m2. Find also the stress in each spoke.

(OR)

25.	A single cylinder double acting steam engine delivers 185 kW at 100 rpm. The maximum fluctuation of energy per revolution is 15 percent of the energy developed per revolution. The speed variation is limited to 1 percent either way from the mean. The mean diameter of the rim is 2.4 m. Design and draw two views of the flywheel.