Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DESIGN OF TRANSMISSION SYSTEMS			            	       Time: 3 hours

Subject Code: 09ME208								       	 	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Distinguish between long bearing and short bearing.

2. 	What are the disadvantages of belt drives when compared to the other drives?

3.	What are the parts of a roller chain?

4.	What is a herringbone gear?

5. 	What is meant by miter gears?

6.	What is virtual number of teeth of a bevel gear?

7.	In what way kinematic arrangement is linked with a speed diagram?

8. 	What are the conditions for selecting the input speeds in a gear box design?

9.	Mention some applications of ratchet and pawl mechanism.

10.	What is the required condition to avoid undercut of roller follower in a cam design?

PART – B  (5 x 3 = 15 MARKS)

11.	Write the mechanism of hydrodynamic lubrication and hydrodynamic bearing.

12. 	List out the advantages of chain drives.

13. 	What kind of contact occurs between worm and worm wheel? How does it differ from other

types of gears?

14.	Briefly explain about the step ratio and structural formula in gear box design.

15.	What are the parts of Geneva mechanism? Mention its applications.

PART – C  (5 x 15 = 75 MARKS)

16.	Design a journal bearing for a steam turbine, whose shaft is supported on two bearings one at each side of the turbine, and is coupled with a generator for power production. The weight of turbine with shaft is measured as 40 kN and the shaft rotates at 1500 rpm. Diameter of the shaft is 100 mm.

(OR)

17. 	Design a V belt drive to transmit 75 kW power from an electrical motor to a machine. The speed of driving wheel is 1400 rpm and the speed of driven wheel is 400 rpm. Diameter of driving wheel is 300 mm and the centre distance is 2500 mm. Take the required service as 16 hours/day.

18.  	Select a wire-rope for a vertical mine hoist to lift a load of 18 kN. The speed of the rope is       	3 m/s and the maximum speed is attained in 10 seconds. The depth of the mine is 500 meters.

(OR)

19. 	Design a pair of helical gears to transmit 10 kW at 1000 rpm of the pinion. Reduction ratio of 5 is required. List out all the dimensions of the gear drive.

20.	Design a bevel gear drive to transmit 7 kW at 1600 rpm and the gear ratio is 3. Take the   	material for pinion and gear as C45 surface hardened.

[P.T.O]

(OR)

21. 	The input to a worm gear shaft is 18,000 Watts and the worm speed is 600 rpm. Velocity  ratio 	is 20:1. An efficiency of at least 85% is desired. Design a worm gear set for the above 	mentioned conditions.

22. 	Design a 12 speed gear if the minimum speed is 31.5 rpm and the maximum speed is          1410 rpm. The gear box receives 5 kW from an electrical motor. The 12 speeds are obtained as

2 X 3 X 2. Sketch the layout of the gear box and the speed diagram.

(OR)

23. 	A 16 speed gear box is to be designed for a machine tool with a step ratio of 1.12. The 	maximum output speed is to be 560 rpm and the minimum speed is 100 rpm. Power to be 	transmitted is 3 kW. Draw the layout of gears and the speed diagram.

24. 	A ratchet and pawl mechanism is used as a brake in a winch and the braking torque is      	150,000 N-mm. The ratchet wheel diameter is 350 mm. Take the allowable bending stress of 	wheel material as 30 N/mm2   and design a suitable ratchet and pawl.

(OR)

25. 	The centre distance between Geneva wheel and driving crank is 60mm and the number of slots 	are 6. Determine the geometric dimensions of the Geneva disc and calculate the power required 	to drive the wheel if the angular velocity is 6.28 rad / sec and the mass moment of inertia of the 	Geneva wheel is 0.1 kg. m2.