Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: RAPID PROTOTYPING								      	      Time: 3 hours

Subject Code: 09ME222			       				               	       	      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What will motivate a design engineer to develop a new product?

2.	What file type is recognized by rapid prototyping (RP) machines?

3.	The main property required for a material to be used in stereo lithography apparatus (SLA) is _______________.

4.	Understanding that the main disadvantage of liquid based processes is being the surface quality, how is the problem solved without modifying the system as such?

5.	Indicate the major process parameter in fused deposition modeling (FDM).

6.	Suggest an alternate method to LASER which is used in laminated object modeling (LOM).

7.	Out of various RP processes which one you will choose for gold jewellery design and making?

8.	Major application of direct shell production casting (DSPC) is ______________.

9.	To manufacture 1,00,000 Teflon washers what kind of rapid tooling you will suggest?

10.	Name one main advantage of using direct metal laser sintering (DMLS) process.

PART – B  (5 x 3 = 15 MARKS)

11.	Assuming that you are a manager in an automobile company, what do you perceive as the key to succeed in the existing competitive market?

12.	Out of SLA and SGC which process will you employ to make a prototype of your conceptual design? Justify your answer.

13.	Is it possible to produce prototypes with different colors? If the answer is yes, which RP process is used for producing color prototypes? State the advantages and disadvantages of the color prototypes.

14.	Derive a method to increase the productivity of the selective laser sintering (SLS) process.

15.	What do you understand by pilot production?

PART – C  (5 x 15 = 75 MARKS)

16.	 Based on your understanding, explain the product development cycle, the current market trend, and the need for a speedy design process.

(OR)

17.	a.	Justify that RP is one of the best ways to reduce the lead time in introducing a product in 	the 	market.																	(5)

b. 	With a flow chart classify the various RP systems. List and explain the general steps 	involved in an RP process.														(10)

18.	Explain with a neat sketch the working principle, process, advantages and disadvantages of SLA Process.

(OR)

19.	Draw the sketches of SLA and SGC processes differentiating their working principle, power source, applications and process parameters.

[P.T.O]

20.	How will you make an aircraft prototype using the FDM process?

(OR)

21.	With a neat sketch explain the LOM process, working principle, advantages, disadvantages and applications.

22.	How can DSPC be used to minimize the lead time and cost in developing a product? Explain its working principle with a neat sketch.

(OR)

23.	Differentiate between SLS and 3D printing technology with respect to machine setup, working principle and application.

24.	Which technology is the future of RP techniques? Based on your observation, what new innovations can be added to the existing RP systems?

(OR)

25.	Write the step by step procedure to develop a silicon rubber mold using the room temperature vulcanizing (RTV) process with a neat sketch. What are the advantages and disadvantages of the RTV process?