Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: METAL CUTTING THEORY AND PRACTICE		              Time: 3 hours

Subject Code: 09ME223			       						              	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the use of a ‘chip breaker’?

2. 	What is the approximate thickness of shear zone in metal cutting?

3. 	What is relief angle of a plain milling process?

4. 	Define up milling.

5. 	List the factor affecting the temperature in machining process.

6. 	Write down the properties of cutting fluids.

7. 	Define tool life.

8. 	List out the requirement of cutting tool materials.

9. 	What is the reason for failure of a cutting tool?

10. 	What are the types of wear?

PART – B  (5 x 3 = 15 MARKS)

11.	What are the assumptions made in Lee and Shaffer’s theory?

12.	List the advantages of strain rings in a milling dynamometer.

13.	What are the functions of cutting fluids?

14.	What are the properties of diamond tool materials?

15.	Define abrasive wear.

PART – C  (5 x 15 = 75 MARKS)

16.	During machining C 25 steel with 0-10-6-7-8-90-1 mm ORS shaped triple carbide cutting tool, the following observations have been made. Feed = 0.18 mm/rev, Depth of cut = 2 mm, Cutting speed = 150 m/min, Fz = 160 kg, Fx = 80 kg, chip thickness = 0.4 mm. Determine:

a.	Chip reduction coefficient.			b.	Shear force and normal force at shear plane.

c.	Kinetic coefficient of friction.		d.	Specific energy of friction.

(OR)

17.	Derive an expression for velocity relationship on metal cutting.

18.	a.	Sketch and explain the various elements of a broaching tool.						(7)

b.	Determine the machining time to drill a hole of 20 mm diameter in a work piece 25 mm thick by a drill, at a cutting speed of 30 meter/min with a feed of 0.2 mm/rev.   		(8)

(OR)

19.	a.	Sketch and explain the nomenclature of a milling cutter.							(7)

b.	Determine the power required to cut a brass bar on a lathe when the cutting speed is                 18 m/min, feed is 0.06 mm/rev and depth of cut is 0.058cm. Assume that the power lost in friction as 30%.     															(8)

20.	a.	Enumerate the expression for temperature in chip formation. 						(7)

b.	Sketch the temperature distribution in machining process. 							(8)

(OR)

21.	Briefly explain the different types of cutting fluids and their applications.

[P.T.O]

22.	Derive an expression for optimum cutting speed for maximum production rate.

(OR)

23.	Derive an expression for tool life for maximum profit.

24.	Briefly explain the wear mechanism in metal cutting.

(OR)

25.	Explain in detail about chatter in machining.