Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MECHATRONICS AND CONTROLS					      	       Time: 3 hours

Subject Code: 09ME224			       					              	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Sketch the graphical representation of a Mechatronics system.

2.	Why is a closed loop control system preferred over an open loop control system?

3.	What is a PLC?

4.	What is a data logger?

5.	Differentiate between PLC and Relay logic control.

6.	What is the main application of a liquid crystal display unit?

7.	State the function of an encoder.

8.	Draw the truth table for an exclusive OR gate.

9.	What is the function of a volume booster?

10.	What is an IP convertor?

PART – B  (5 x 3 = 15 MARKS)

11.	State the differences between the conventional and mechatronics design.

12.	What is the function of a master control?

13.	What are the applications of CRO?

14.	List the functions of a back pressure sensor.

15.	Sketch and label the resolver.

PART – C  (5 x 15 = 75 MARKS)

16.	Compare and contrast the traditional design of an SLR camera and a digital SLR camera.

(OR)

17.	a.	With an example, explain the various functional units of a measurement system.

b.	Discuss the application of a closed loop control of a temperature monitoring system in a room.

18.	Explain the architecture of PLC with a detailed block diagram.

(OR)

19.	Derive the equation for Rouths and Hourwitz stability criteria.

20.	With a simple circuit, explain the constructional features of analog and digital indicators.

(OR)

21.	Discuss about the data acquisition system used for online pressure and flow control of liquid through a pipe.

22.	With an aid of a neat schematic, describe the working of a stepper motor and its characteristic curves.

(OR)

23.	With a neat diagram, discuss the fluidic logic gates.

24.  	Describe an air relay and force balance system with suitable diagrams.

(OR)

25.	Sketch and explain the circuit for a PID pneumatic control and its application in process control.