Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: HEAT ENGINES AND FLUID MACHINERY			      	       Time: 3 hours

Subject Code: 09ME228						       		             	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define density.

2.	Differentiate between single acting and double acting reciprocating pump.

3.	Define the term turbine.

4.	What is a draft tube?

5.	What is a thermodynamic system?

6.	Define Carnot’s theorem.

7.	Draw the p-v diagram for an Otto cycle.

8.	List the four processes of the Diesel cycle.

9.	What is Fourier’s law of heat conduction?

10.	What is a black body?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the operating characteristics curves of a centrifugal pump.

12.	How will you classify the turbines?

13.	List the different types of equilibrium.

14.	State the four processes of Brayton cycle.

15.	How is fin efficiency defined?

PART – C  (5 x 15 = 75 MARKS)

16.	The outer diameter of an impeller of a centrifugal pump is 400 mm and outlet width is 50 mm. The pump is running at 800 r.p.m. and is working against a total head of 15 m. The vanes angle at outlet is 40˚ and manometric efficiency is 75%. Determine: 	a.	Velocity of flow at outlet

b.	Velocity of water leaving the vane		c.	Angle made by the absolute velocity at outlet with the direction of motion at outlet	d.	Discharge.

(OR)

17.	List the comparison between centrifugal pump and reciprocating pump.

18.	A Pelton wheel has a mean bucket speed of 10 m/sec with a jet of water flowing at the rate of 700 liters/sec under a head of 30 meters. The buckets deflect the jet through an angle of 160˚. Calculate the power given by water to the runner and the hydraulic efficiency of the turbine. Assume co-efficient of velocity as 0.98.

(OR)

19.	Explain the governing mechanism of a Pelton turbine with a neat sketch.

20.	0.15m3 of air has a temperature and pressure of 800 K and 600 kN/m2 respectively. It is expanded polytropically to a pressure of 200 kN/m2. If the index of expansion n = 1.3, calculate a.	The amount of heat transfer	b.	Change in internal energy 		c.	The work done.

(OR)

[P.T.O]

21.	A certain water heater operates under steady flow conditions receiving 4.2 kg/sec of water at 	75˚C temperature, enthalpy 313.93 kJ/kg. The water is heated by mixing with steam which is 	supplied to the heater at a temperature 100.2˚C and enthalpy 2676 kJ/kg. The mixture leaves 	the 	heater as liquid water at the temperature 100˚C and enthalpy 419 kJ/kg. How much steam 	must be supplied to the heater per hour?

22.	Explain briefly Otto cycle with help of p-v and T-S diagram, and derive an expression for the ideal efficiency of Otto cycle.

(OR)

23.	In an ideal Brayton cycle, air from the atmosphere at 1 atm., 300 K is compressed to 6 atm. and the maximum cycle temperature is limited to 1100 K by using a large air-fuel ratio. If the heat supply is 100 MW, find:	a.	the thermal efficiency of the cycle		b.	work ratio

c.	power output		d. 	energy flow rate of the exhaust gas leaving the turbine.

24.	Show that, for estimating radial heat conduction through a cylindrical wall, the log-mean area of the inner and outer surfaces has to be considered.

(OR)

25.	A body at 1000˚C in black surroundings at 500˚C has an emissivity of 0.42 at 1000˚C and an emissivity of 0.72 at 500˚C. Calculate the rate of heat loss by radiation per m2.

a.	When the body is assumed to be grey with є = 0.42.

b.	When the body is not grey.