Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MECHATRONICS										       Time: 3 hours

Subject Code: 09ME253			       				               	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Draw the block diagram of a measurement system and its constituent elements.

2.	What is the main drawback with pneumatic systems?

3.	Define the term transducer.

4.	What is meant by the term ‘Reproducibility’?

5.	Define interfacing.

6.	What is a solenoid?

7.	List the various parts of microprocessor systems.

8.	Expand the following abbreviations: ADC, DAC, SCADA and HMI.

9.	Define PLC.

10.	What are the basic symbols used in ladder logic programming.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the reasons for automation?

12.	How will you classify sensors?

13.	Draw an interfacing circuit for a seven-segment display with a decoder.

14.	What are the factors to be considered for the selection of a microcontroller?

15.	What are the advantages and disadvantages of PLCs over computers?

PART – C  (5 x 15 = 75 MARKS)

16.	Identify the various elements that might be present in a control system involving a thermostatically controlled electric heater for a room.

(OR)

17.	Design a pneumatic circuit for a two actuator sequential operation of cylinders.

18.	Explain the working of LVDT and its output characteristics with the aid of neat sketches.

(OR)

19.	Write short notes on:																(3x5)

a.	Pyro elecric sensors		b.	Tactile sensors		c.	Light sensors

20.	a.	Discuss in detail about sensor interfacing and buffer ICs.							(6)

b.	Explain the construction and working principle of a DC motor.						(9)

(OR)

21.	a.	How does an electromagnetic relay operate?										(5)

b.	Describe the relay controlled system for pneumatic valves to control the movements of 	three cylinders.																(10)

22.	Draw the block diagram of a microcontroller and explain the function of each subsystem.

(OR)

23.	a.	Give a detailed account on pin diagram of Intel 8051.								(10)

b.	Draw the block diagram of a temperature measurement system using a MC68HC11.	(5)

[P.T.O]

24.	a.	Explain briefly the architecture of a Programmable Logic Controller.				(5)

b.	Discuss various logic combinations of switches and ladder programs for them.		(10)

(OR)

25.	Write notes on the following:														(7+8)

a. 	Motor Latching 		         	b. 	Cylinder Sequencing