Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: APPLIED THERMODYNAMICS AND HEAT TRANSFER         Time: 3 hours

Subject Code: 09ME256				       				                	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The temperature at which the volume of a gas becomes zero is called __________.

2.	Difference between the pressure of a fluid and the pressure of atmosphere is called as __________.

3.	For any irreversible process the net entropy change is __________.

4.	C.O.P. of a reversible heat pump is 1.2. If it is reversed to run as reversible heat engine then its 	efficiency shall be __________.

5.	Joule’s law states that the specific internal energy of a gas depends only on the __________    of the gas.

6.	__________ results in increase of COP provided that no further energy has to be spent to obtain the extra cold coolant required.

7.	The __________ of heat transfer by convection depends largely upon the mixing motion of the 	fluid.

8.	The ability of a surface of a body to radiate heat is called as __________.

9.	A Condenser generally operates at __________ pressure.

10.	The ratio of sum of actual output of stages and the available energy in a turbine is called __________.

PART – B  (5 x 3 = 15 MARKS)

11. 	Define the following: 		a. 	Fahrenheit scale		b.	Rankine scale.

12. 	Give the conditions of the process to be reversible.

13. 	Name four important properties of a good refrigerant.

14. 	What is sub-cooling? What is its effect on the COP of a vapour compression system?

15. 	Differentiate between petrol and diesel engines.

PART – C  (5 x 15 = 75 MARKS)

16. Explain the working principle of pressure thermometers with a suitable sketch.

(OR)

17. 	A system comprising of a gas of 5 kg mass undergoes expansion process from 1 MPa and       0.5 m3 to 0.5 MPa. Expansion process is governed by, p.v1.3 = constant. The internal energy of gas is given by, u = 1.8 pv + 85, kJ/kg. Here ‘u’ is specific internal energy, ‘p’ is pressure in kPa, ‘v’ is specific volume in m3/kg. Determine heat and work interaction and change in internal energy.

[P.T.O]

18. 	The following equation gives the internal energy of a certain substance	   u = 3.64 pv + 90

where u is kJ/kg, p is in kPa and v is in m3/kg. A system composed of 3.5 kg of this substance expands from an initial pressure of 500 kPa and a volume of 0.25 m3 to a final pressure 100 kPa in a process in which pressure and volume are related by pv1.25 = constant.

a.	If the expansion is quasi-static, find Q, ΔU and W for the process.

- In another process, the same system expands according to the same pressure-volume 	relationship as in part ‘a’, and from the same initial state to the same final state as in part 	‘a’, but the heat transfer in this case is 32 kJ. Find the work transfer for this process.

(OR)

19. 	A reversible heat engine operates between two reservoirs at temperatures 700°C and 50°C. The engine drives a reversible refrigerator which operates between reservoirs at temperatures of 50°C and – 25°C. The heat transfer to the engine is 2500 kJ and the net work output of the combined engine refrigerator plant is 400 kJ.

a.	Determine the heat transfer to the refrigerant and the net heat transfer to the reservoir at 50°C.

b.	Reconsider part ‘a’, given that the efficiency of the heat engine and the C.O.P. of the 	refrigerator are each 45 per cent of their maximum possible values.

20. 	A vessel having a capacity of 0.05 m3 contains a mixture of saturated water and saturated steam at a temperature of 245°C. The mass of the liquid present is 10 kg. Find the following:

a.	The pressure.

b.	The mass.

c.	The specific volume.

d.	The specific enthalpy.

e.	The specific entropy.

f.	The specific internal energy.

(OR)

21. 	Draw and explain a p-T (pressure-temperature) diagram for a pure substance.

22. 	A 150 mm steam pipe has an inside diameter of 120 mm and outside diameter of 160 mm. It is insulated at the outside with asbestos. The steam temperature is 150°C and the air temperature is 20°C. h (steam side) = 100 W/m2°C, h (air side) = 30 W/m2°C, k (asbestos) = 0.8 W/m°C and k (steel) = 42 W/m°C. How thick should the asbestos be provided in order to limit the heat losses to 2.1 kW/m2?

(OR)

23. 	Calculate the following for an industrial furnace in the form of a black body and emitting radiation of 2500°C:

a.	Monochromatic emissive power at 1.2 μm length.

b.	Wavelength at which the emission is maximum.

c.	Maximum emissive power.

d.	Total emissive power.

e.	Total emissive power of the furnace if it is assumed as a real surface with emissivity equal 		to 0.9.

24. 	Explain the working of a Cochran boiler with a neat diagram.

(OR)

25. 	Discuss the working of a summer air-conditioning system with a neat diagram.