Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: AUDIO ENGINEERING								       	      Time: 3 hours

Subject Code: 09VC209			       				             		       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Name any two devices which can connect between real sound and an electrical equivalent.

2.	Which two specific characteristics of sound are used in microphone directional pickup pattern?

3.	In a sound mixer what are PFL and AFL?

4.	Why is the purpose of a graphic equalizer?

5.	In magnetic tape recording, what is "de-gauzing"?

6.	Expand  - ADAT.

7.	Express 'threshold of pain' in dB.

8.	List out any two drawbacks of FOH position.

9.	Expand Tascam 'TDIF'.

10.	Expand the audio driver format-ASIO.

PART – B (5 x 3 = 15 MARKS)

11.	Why is Phantom powering required in microphones?

12.	Distinguish between compressors and limiters.

13.	In analog tape recording, explain system noise and tape noise.

14.	Why is (controlling audio) live music in outdoors more difficult?

15.	In digital audio, explain 'word clock'.

PART – C (5 x 15 = 75 MARKS)

16.	What constitutes an ideal loud speaker? Discuss the following loud speaker concepts:

a.	Sound Pressure Level and volume velocity.

b.	Lowest frequencies must have large diaphragm.

c.	More number of drive units: woofer, tweeters, mid range or squakers.

d.	Cross over network.

e.	Loud speaker enclosure functions.

(OR)

17.	What is the need for monitoring? Explain stereophony and surround sound.

18.	List out any ten basic functions of a sound desk.

(OR)

19.	Why is the sound of effects and equalization too hard to translate into engineering terms? Define the following to the nearest possible explanation:

a.	constricted 		b.	edgy 	c.	airy 	d.	muffled 		e.	punchy

20.	Without any reference to digital audio list out any five advantages of analog recording and storage.

(OR)

21.	List out any TEN advantages of digital audio recording over analog.

[P.T.O]

22.	Discuss two microphone placements, recording from performing groups' PA mixer for outdoor simple music recording.

(OR)

23.	Draw a block diagram for a large on-location music recording venue, starting from input and ending with output.

24.	Explain digital recording by pulse code modulation with suitable drawings showing waveform and its conversion.

(OR)

25.	Describe the digital audio workstation along with five features of HD recorder-mixer.