Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ELECTRIC CIRCUITS AND NETWORKS				      	       Time: 3 hours

Subject Code: 10EE201						       		        	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Obtain the equivalent resistance of two 60 Ohm resistors in parallel.

2.	State Kirchoff’s current law.

3.	Another name of the substitution theorem is __________ .

4.	Write the most common application of Maximum power transfer theorem

5.	A 25 Ohm resistance has a voltage v = 150.0 sin 377 t (V). Find the corresponding current i.

6.	What is zero state response?

7.	Write down the equation to measure the 3 phase power using two-watt meters.

8.	Define coefficient of coupling.

9.	Write the condition for symmetry in terms of Z parameters.

10.	What is an Ideal Filter?

PART – B  (5 x 3 = 15 MARKS)

11.	Find the voltage Vab in the network shown below.

12.	State the Substitution Theorem and explain.

13.	A DC voltage of 100 Volts is applied to a series RL circuit with R = 25 Ohms. What will be the current in the circuit at twice the time constant?

14.	Two identical coupled coils in series have an equivalent inductance value of 0.084 H and 0.0354 H. Find the values of the Self and Mutual Inductances.

15.	On what factors are Electric Wave filters classified?

PART – C  (5 x 15 = 75 MARKS)

16.	For the AC network shown in the figure given below determine, using mesh-current analysis,       a.	the mesh currents I1 and I2 	b.	the current flowing in the capacitor		c.	the active power delivered by the 100 𝘓0° V voltage source.

[P.T.O]

(OR)

17.	For the network shown in the figure given below, determine the voltage VAB, by using nodal analysis

18.	Replace the network given below at terminals ab with a Norton’s equivalent and with a Thevenin’s equivalent.

(OR)

19.	Find the load impedance in the figure given below for maximum power to the load, 	and find the maximum power.

20. 	A 500 nF capacitor is connected in series with a 100 k resistor and the circuit is connected to a 50 V, DC supply. Calculate:		a.	the initial value of current flowing		b.	the value of current 150 ms after connection		c.	the value of capacitor voltage 80 ms after connection		d.	the time after connection when the resistor voltage is 35 V.

(OR)

21. 	With a suitable example explain the following:

a.	Initial value theorem

b.	Final value theorem.

22.	For the unbalanced Delta-connected load of the figure given below with two properly connected watt meters determine the following:

[P.T.O]

a.	The magnitude and angle of the phase currents.

b.	The magnitude and angle of the line currents.

c.	The power reading of each wattmeter.

d.	The total power absorbed by the load.

(OR)

23. 	Derive the voltage equation for the coupled circuit given below.

24.	A two port network has the following open circuit and short circuit impedances Z10 = 900 Ω,  Z20 = 1000 Ω  Z1s = 650 Ω . Find the parameters of the T equivalent circuit to represent the two port network.

(OR)

25.	Design a constant k LPF to cut off at 1000 Hz and a load of 500 Ω. Calculate the attenuation at 1500 Hz and phase shift at 50 Hz.