Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: CIRCUIT ANALYSIS AND NETWORKS				       	       Time: 3 hours

Subject Code: 10EI201			       						             	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	If 90 J of energy is available for every 30 C of charge, what is the voltage?

2.	In a parallel circuit, the current ____________ in all branches.

3.	Name any one application of superposition theorem.

4.	Thevenin’s equivalent circuit consists of ___________________.

5.	An Inductor does not allow sudden changes in _____________.

6.	Transient current in an RLC circuit is oscillatory when __________.

7.	Dot convention in coupled circuits is used to determine the ________ of the mutually induced voltage in coils.

8.	The maximum possible mutual inductance of two inductively coupled coils with self inductances L1=25 mH and L2=100 mH is given by __________.

9.	The number of possible combinations generated by four variables taken two at a time in a two port network is _________.

10.	An ideal filter should have _______ attenuation in the pass band.

PART – B (5 x 3 = 15 MARKS)

11.	State Kirchoff’s laws.

12.	State Norton’s theorem.

13.	For the given network function draw the pole zero diagram and hence obtain the time domain response.  I(S)=3s(s+1)(s+3).

14.	What is coefficient of coupling?

15.	What is the propagation constant of a T - network?

PART – C (5 x 15 = 75 MARKS)

16. 	Determine the value of resistance R and current in each branch when the total current taken by the circuit shown below is 6 A.

(OR)

[P.T.O]

17.	Write the node voltage equations and determine the currents in each branch for the network shown below,

18.	Determine the Thevenin’s equivalent circuit of the network given below. Also find the current through a 5 Ω galvanometer connected across AB.

(OR)

19.	Determine the load resistance to receive maximum power from the source. Also find the maximum power delivered to load in the circuit.

20.	Derive the expression for transient response i(t) for an R-C circuit with a DC input. Also find the transient voltage across the inductor and resistor.

(OR)

21.	The circuit shown below consisting of series RLC elements with R=10 Ω, L=0.5 H and C=200 µF has a sinusoidal voltage v(t)=150sin(200t+Φ). If the switch is closed when Φ=30°, determine the current equation.

22.	Derive the maximum output voltage for a single tuned circuit. Also plot the variation of amplification factor with frequency for different coupling coefficients.

[P.T.O]

(OR)

23.	a.	A voltage v(t)=10sinωt is applied to a series RLC circuit. At the resonant frequency of 	the circuit, the maximum voltage across the capacitor is found to be 500 V. Moreover, the 	bandwidth is known to be 400 rad/sec and the impedance at resonance is 100 Ω. Find the 	resonant frequency. Also find the values of L and C of the circuit.             			(8)

b.	A series RLC circuit has a quality factor of 5 at 50 rad/sec. The current flowing through the 	circuit at resonance is 10 A and the supply voltage is 100 V. The total impedance of the 	circuit is 20 Ω. Find the circuit constants.                    								(7)

24.	The Z parameter of a two-port network are Z11=10Ω; Z22=15Ω; Z12=Z21=5Ω. Find the equivalent T network and ABCD parameters.

(OR)

25.	Design an m-derived low pass filter having cut-off frequency of 1kHz, design impedance of 400Ω and a resonant frequency 1100 Hz.