Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ELECTRONIC CIRCUITS								      	       Time: 3 hours

Subject Code: 10EI202							       		    		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The general current equation of a PN junction diode is_________.

2.	The transformer utilization factor of a half wave rectifier is___________.

3.	The h-parameters are valid over the ____________ frequency range.

4.	Expression for β* is______________.

5.	The formula for identifying the harmonic distortion is ___________.

6.	What is theoretical maximum conversion efficiency of class A transformer coupled power amplifier?

7.	Two types of N-channel and P-channel MOSFETs are __________ and ___________.

8.	Draw the small signal model of FET.

9.	An oscillator circuit in which the reactance α1, α2 are capacitive and α3 is inductive is called ___________.

10.	The expression for densitivity of an amplifier circuit with negative feedback is __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define regulation in a rectifier.

12.	Define load line with its significance.

13.	With its merits and demerits discuss how the power amplifiers are classified.

14.	What is cross over distortion? How can it be eliminated?

15.	Mention the types of feedback oscillators.

PART – C  (5 x 15 = 75 MARKS)

16.	A diode whose internal resistance is 20 Ω is to supply power to a 1000 Ω load from a             110 V (rms) source of supply. Calculate 	a.	The peak load current 		b.	The DC load current 		c.	AC Load Current 	d.	The DC diode voltage 	e.	The total input power to the circuit 		f.	% regulation from no load to the given load.

(OR)

17.	Explain the operation of a bridge rectifier with a suitable diagram and compare the performance of a bridge rectifier with a half and a full wave rectifier.

18.	With a neat circuit diagram, derive the generalized expression for analyzing the operation of an amplifier circuit using h-parameter model.

(OR)

19.	Discuss the following in detail:

a.	For any transistor amplifier prove that:

Ri =hi / (1–hrAv)

b.	Cascade Amplifier.

20.	Prove that the maximum efficiency of a class A transformer coupled power amplifier is 50% 	and that of class B type 78.5%.

[P.T.O]

(OR)

21.	Draw the circuit diagram of an RC coupled amplifier and discuss the frequency and phase response characteristics.

22.	List down the different types of FET biasing circuits and explain them in detail.

(OR)

23.	Draw and explain the small signal model of a FET amplifier.

24.	With a neat sketch, derive the expression for Aol, Acl, Ri, Ro, gain and bandwidth of a voltage series feedback amplifier.

(OR)

25.	Draw and explain the Wien bridge oscillator circuit. Also derive the expression for frequency of oscillation.