Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: SIGNALS AND SYSTEMS								       	      Time: 3 hours

Subject Code: 10EI204			       					             		       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Represent the unit step function in terms of unit impulse function.

2.	State the linearity property of a system.

3.	If N number of samples are convolved with M number of the samples, then the resultant signal will have ______ number samples.

4.	Write down the difference equation of a first order DT system.

5.	If the function x(t) is an odd function then the complex exponential Fourier series coefficient is a.	purely real 		b.	purely imaginary 		c.	purely complex.

6.	Compute the power for the nth real harmonic of x(t) using the complex Fourier series coefficient Cn..

7.	State the time shifting property of DFS.

8.	Compute the DTFT of the given sequence x(n) = {1,1,1}.

9.	Obtain the ROC of the given causal sequence h(n) = {1,2,3,4}.

10.	Derive the transfer function of the given first order system y(n) = x(n) – 0.9y(n1).

PART – B (5 x 3 = 15 MARKS)

11.	For the given signal x(n) = {1,2,3,4,5,6}, compute x(2n-1) and x(n/2).

12.	Find the Convolution of the signals x(n) = {1,2,1,1} and h(n) = {1, 1, 1, 1} where bold   represents the origin.

13.	State and prove Parseval’s relation for the given sequence x(n) = {1,1,1,1}.

14.	Find the discrete time Fourier transform of the given sequence x(n) = nan u(n).

15.	Check the stability of the system whose impulse response h(n) = 2nu (n).

PART – C (5 x 15 = 75 MARKS)

16.	a.	Determine the even and odd components of x(t) = cost+sin t.					(4)

b.	Compute the energy and power of the given signal x(t) = e-3|t|						(4)

c.	A discrete time signal x(n) = {0,0,1,1,2}. Compute: 	i.	 x(2n) 	ii. 	x(n)

iii.	x(n+2)	iv.	x (n+2)		v.	x(n2).										(7)

(OR)

17.	A digital system is characterized by the difference equation:

y(n) = 0.5x(n) 0.25 x(n1) + 0.75 y(n1). Check if the system is 		a.	Static or dynamic

b.	Linear	c.	Causal	d.	Time variant  or  invariant		e.	Stable.

Give suitable reasons for your answer.

18.	Determine the transfer function, unit impulse response and unit step response of the RC network shown in figure below using convolution integral.

[P.T.O]

(OR)

19.	Consider a causal, stable LTI system whose input x(n) and output y(n) are related by the second order difference equation:

y(n) – (1/12) y(n1) – (1/12) y(n2) = x(n).

Determine the impulse response and unit step response of the given system.

20.	a.	Determine the exponential Fourier series coefficient Cn of the given periodic signal. 	(9)

x(t) = −1 −T2<t<01 0≤t≤T2

b. 	Determine the spectrum of x(t) = cos ot by using frequency shifting property.		(6)

(OR)

21.	a.	Compute the Fourier transform of a rectangular pulse x(t) =1 for 0≤t≤T0 otherwise		(7)

b.	Derive the frequency response the given system characterized by the differential equation:

d2y(t)dt2−0.5dy(t)dt+0.4y(t) = x(t).												(8)

22.	a.	State and prove any four properties of DTFT.									(8)

b.	Find the DTFT of the given signal	x(n) = A for 0≤n≤N−10 otherwise					(7)

(OR)

23.	a.	The output response y(n) of an LTI system to the input x(n) is  y(n) = x(n) – 2x(n1) + 		x(n+2). Compute and sketch the magnitude and phase of the frequency response of the 	system.																		(10)

b.	State and prove the Sampling theorem.											(5)

24.	a.	The transfer function of the network is   H(s) =1RC.1s+1RC.  Compute the output for 	the 	input  x(t)=te-t/RC	.														(7)

b.	State and prove any four properties of Z transform.								(8)

(OR)

25.	a.	Find the Z transform  x(n)=(13)nu(n)+(−2)nu(−n).  							(4)

b.	Determine the inverse Z transform of X(z) =z3z2−4z+1 for 13<z<1.	  		(4)

c.	Compute the impulse response of the system described by the difference equation:

y(n) = 0.6y(n1) 0.08y(n2)+x(n).											(7)