Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: CONTROL SYSTEMS										       Time: 3 hours

Subject Code: 10EI205			       					             		       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What are the two major types of control systems?

2.	Define transfer function.

3.	What is the need for a controller?

4.	Mention different types of stepper motors.

5.	The number of open-loop poles at the origin is called the_________ of the system.

6.	Define settling time.

7.	What is frequency response?

8.	Draw the polar plot of

9.	The path taken by the roots of the characteristic equation when the open loop gain K is varied from 0 to  is called _________.

10.	Define asymptotic stability.

PART – B (5 x 3 = 15 MARKS)

11.	Consider the two masses-spring system shown in the figure below. Determine X2(s)F(s).

12.	What is a Gyroscope? What are its applications?

13.	The open-loop transfer function of a unity feedback servo system is given 	by:G(s)=10s(0.1s+1).Evaluate the static error constants for the system.

14.	The damping ratio and natural frequency of oscillation of a second order system is 0.5 and        8 rad/sec respectively. Calculate the resonant peak and resonant frequency.

15.	Using Routh criterion, determine the stability of the system represented by the characteristic equation

PART – C (5 x 15 = 75 MARKS)

16.	Using block diagram algebra, obtain the closed-loop transfer function for the block diagram shown in the figure below.

[P.T.O]

(OR)

17.	Find the closed-loop transfer function of the system whose signal flow graph is shown in the figure below, using Mason’s gain formula.

18.	Explain the use of synchro pair as an error detector.

(OR)

19.	Compare the armature and field controlled DC servomotors. Derive the transfer function of a field controlled DC servo motor.

20.	The open-loop transfer function of a unity feedback system is given by:

G(s)=Ks(s+10).

Determine the gain K so that the system will have a damping ratio of 0.5. For this value of  determine settling time, peak overshoot and time to peak overshoot for a unit-step input.

(OR)

21.	Discuss the effect of P controller and PI controller on the system performance.

22.	Construct Bode plot for a unity feedback system having: G(s)=10s(0.4s+1)(0.1s+1).	Determine the gain and phase cross over frequencies.

(OR)

23.	Sketch the Nyquist plot for the open-loop transfer function: G(s)H(s)=6(s+1)(s+2).Determine 	the stability of the closed-loop system by Nyquist criterion.

24.	Sketch the root locus of the system whose open loop transfer function is given by:

G(s)=Ks(s+2)(s+4).

(OR)

25.	State Routh Hurwitz stability criterion. For the system having characteristic equation:  find the following using Routh Hurwitz stability criterion:

a.	the number of roots in the left half of s-plane.

b.	the number of roots in the right half of s-plane.

c.	the number of roots on the imaginary axis.