Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: SIGNAL CONDITIONING CIRCUITS						       Time: 3 hours

Subject Code: 10EI206		       						             		       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The open loop gain of an ideal non-inverting amplifier is _____________.

2.	The Input bias current value of BJT is___________.

3.	The Schmitt trigger is otherwise called as ____________.

4.	State the expression for the duty cycle and the frequency of oscillations for an astable circuit using IC555.

5.	The gain equation of a two stage instrumentation amplifier is _____________.

6.	The frequency equation of a high pass filter is ___________.

7.	Define the scale factor of a multiplier.

8.	Mention the different types of voltage regulators.

9.	A PLL has a free running frequency of 500 KHz and bandwidth of LPF is 10 KHz. Will the loop acquire lock for an input signal of 600 KHz? Justify the answer. Assume the phase detector produces sum and difference of frequency components.

10.	List the two different types of digital phase detectors used in PLL.

PART – B (5 x 3 = 15 MARKS)

11.	Write down the slew rate equation and discuss its effects.

12.	With a neat sketch and necessary equation discuss grounded type V-I converter.

13.	Draw the circuit diagram of first order low pass and high pass filter.

14.	With a neat sketch, justify how the voltage is divided using multiplier IC.

15.	Define capture and lock-in range.

PART – C (5 x 15 = 75 MARKS)

16.	Define frequency response. Discuss with necessary equations how the frequency response varies with gain.

(OR)

17.	List down the different DC characteristics of Op-Amps and explain any three of them in detail.

18.	Design an Integrator and differentiator using Op-Amps and analyze their output for various input functions.

(OR)

19.	Draw and explain the astable mode of operation of IC 555 timer. Derive the expression for frequency of oscillation.

20.	Explain the design of a two stage and three stage instrumentation amplifier using Op-Amp.

(OR)

21.	With necessary equations justify the functioning of low pass and high pass filter.

22.	Discuss how the multiplier circuit performs the square root and frequency doubling operations.

(OR)

[P.T.O]

23.	What are the features of LM723 Regulator? Describe the various protection circuits used for series voltage regulator and explain them in detail.

24.	Explain the working of Voltage controlled oscillator with necessary circuit diagrams and derive the expression for Kv.

(OR)

25.	Draw and explain the operation of PLL with its various operating regions and its applications.