Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: PROCESS DYNAMICS AND CONTROL			      	      	       Time: 3 hours

Subject Code: 10EI208						       		        	      	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is the need for process control?

2.	Define time constant of a process.

3.	List the limitations of On-Off control mode.

4.	Write the Transfer Function of Proportional Integral Control.

5.	List any two tuning methods of a controller.

6.	What is ITAE?

7.	Define the flow coefficient of a control valve.

8.	Write about any two factors involved in the selection of a control valve.

9.	What is meant by auctioneering control?

10.	Mention the principle involved in heat exchangers.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Self Regulation with an example.

12.	Why derivative control is not recommended alone for any process?

13.	Write a brief note on ¼ decay ratio.

14.	What is the need for a Valve Positioner?

15.	What is a Reboiler?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the term degrees of freedom of a process.								(5)

b.	Derive the mathematical model for a liquid level process with constant flow outlet. 	(10)

(OR)

17.	a.	Differentiate continuous and batch process with a suitable example.				(5)

b.	Derive the transfer function for the interacting tank system with necessary diagrams.	(10)

18. 	Bring out the relative advantages and disadvantages of proportional, integral and derivative 	control actions.

(OR)

19. 	a.	What is Integral wind up? How can it be prevented?								(5)

b.	Explain in detail the selection of control modes for level and temperature process.	(10)

20. 	Explain the process reaction curve method of tuning a PID controller.

(OR)

21. 	Describe the Ziegler-Nichols tuning methodology. This procedure is often called as continuous

cycling tuning method. Why?

22. 	a.	Describe the construction of a control valve.										(8)

b.	Explain the three different characteristics of a control valve.						(7)

(OR)

[P.T.O]

23. 	a.	Explain the operation and benefits of a Valve Positioner.							(8)

b.	Discuss about Cavitation and Flashing in control valves.							(7)

24. 	Write short notes on:

a.	Ratio Control.																(8)

b.	Split Range Control.															(7)

(OR)

25. 	a.	When should one go for a feed forward control? Give an example for the same from 	a			distillation column. 															(8)

b.	Explain how cascade control can give better performance than single loop controller with

an example.																	(7)