Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DIGITAL CONTROL SYSTEMS							       Time: 3 hours

Subject Code: 10EI213			       					             		       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define quantization.

2.	What is meant by periodic sampling?

3.	Give the z-transform for a unit step signal.

4.	Mention any two methods of obtaining inverse z-transformation.

5.	Define pulse transfer function.

6.	Define the term observability.

7.	For the system to be stable, the closed loop poles must lie within the unit circle in the z-plane – 	True or False?

8.	The bilinear transformation is performed by substituting __________ in the characteristic equation.

9.	Poles near z = -1 are often referred to as __________ poles.

10.	When the input to a discrete time system is unit impulse, the output is called impulse response - True or False?

PART – B (5 x 3 = 15 MARKS)

11.	Draw the block diagram of a sampled data control system.

12.	Obtain the modified z-transform of G(s), where

G(s)=1(s+a).

13.	Consider the system defined by G(z)=z+1z2+z+0.16.

Obtain the state-space representation for this system in controllable canonical form and observable canonical form.

14.	How many rows are formed in Jury’s table and what are the sufficient conditions to be checked from this table for stability?

15.	Mention the properties of deadbeat response.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Explain the sampling process in frequency domain and Shannon’s sampling theorem.	      																					(8)

b.	Write a brief note on the reconstruction of sampled signals using ZOH circuit.	  	(7)

(OR)

17.	a.	Show that the transfer function of ZOH is given by Gho(s) =1−e−sTs.	       			(7)

b.	Explain the operation of ramp type A/D converter.					         		(8)

[P.T.O]

18.	a.	Find the inverse z transform of	X(z)=z+2z2(z−2).								(8)

b.	A unity feedback system is characterized by an open-loop transfer function GhoG(z) =0.2385(z+0.876)(z−1)(z−0.2644)

Determine the steady state error for unit step, ramp, and acceleration inputs.		       (7)

(OR)

19.	a.	Find the z-transform of a discrete-time system for unit ramp input.      			       (7)

b.	A discrete-time system is described by the transfer function. G(z)=Y(z)R(z)=0.05zz−0.95

Find the response y (k) of the system to unit step input.		       			         	(8)

20.	a.	Obtain the block diagram for the following pulse transfer function by direct programming.     	G(z)=Y(z)X(z)=2−0.6z−11+0.5z−1														(7)

b.	Obtain the z-transform of the system output for the block diagram shown in the figure 			below.																		(8)

(OR)

21.	Consider the system described by

Determine a suitable state feedback gain matrix K such that the system will have the characteristic roots at 0.6±j0.4.

22.	Using Jury’s stability test, check if all the roots of the following characteristic equation lie within the unit circle. Comment on the stability of the system. z3−1.1z2−0.1z+0.2=0.

(OR)

23.	Describe the mapping between the s-plane and the z-plane.

24.	A PID controller is described by the following relation between input e(t) and output u(t):

u(t)=Ke(t)+1Ti0te(t)dt+TDde(t)dt

Discretize the above equation using backward-difference approximation for the derivative term and trapezoidal summation for the integral term. Also obtain the pulse transfer function U(z)/E(z).

(OR)

25.	Draw the block diagram of digital position control system and explain.