Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: COMMUNICATION ENGINEERING					      	       Time: 3 hours

Subject Code: 10EI214						       		        	      	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is meant by modulation index in AM?

2.	Find the bandwidth of AM if Fc=2MHz and Fm=10KHz.

3.	Mention the IF for standard commercial AM.

4.	Define Thermal Noise

5.	List any two merits of PCM.

6.	Determine the peak frequency deviation for binary FSK signal with mark frequency 49 KHz, space frequency 51KHZ and input bit rate 2Kbps.

7.	What is the basic principle of optical communication?

8.	List the merits of TDM

9.	What is a passive satellite?

10.	What is the need for a MODEM?

PART – B  (5 x 3 = 15 MARKS)

11.	The input to conventional AM modulator is 500 KHz carrier with an amplitude 20V peak value and 10 KHz modulating signal with peak value 7.5V. Write the expression for the AM wave.

12.	What are the types of Noise?

13.	Draw the block diagram of a differential PCM system.

14.	Explain the construction of a hetero junction LED.

15.	What are factors that affect radar range?

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	What is the need for modulation?												(5)

b.	Derive the expression for Amplitude Modulation.								(10)

(OR)

17. 	With a neat block diagram, explain the generation of DSB-SC wave and SSB wave.

18.  What are basic building blocks of an AM transmitter? Explain the function of each block in 	detail.

(OR)

19.	a. 	Discuss the operation of any one type of FM detector.

b. 	Compare the effect of noise in AM and FM.

20. 	a. 	Explain PAM generation and discuss its merits and demerits.						(10)

b. 	Compare PCM and differential PCM.											(5)

(OR)

21. Determine the signal to noise ratio required according to the Shannon’s limit to transmit a 	multiplexed telephone signal at 8.448Mbps through a channel of 2.048MHz bandwidth. If this 	8.448Mbps signal needs to be transmitted through 2.048MHz bandwidth channel using M-ary 	encoding calculate M and the number of bits to be encoded as a single symbol.

[P.T.O]

22.	Draw the fiber optic complete system and discuss the function of various blocks. Mention the merits and demerits.

(OR)

23. 	Explain the following:

a.	Error detection and correction.

b.	Multiplexing.

24. 	Discuss with a neat diagram, the working of a TV receiver and a Facsimile system

(OR)

25.  Write brief notes on:

a.	RS 232 operation.

b.	Cellular Communication.