Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MODERN CONTROL SYSTEMS						      	       Time: 3 hours

Subject Code: 10EI218								     		    	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define gain of a system.

2.	What are the components of rotational mechanical systems?

3.	Write the equation for settling time.

4.	Define steady state error.

5.	List the frequency domain specifications.

6.	What is gain cross over frequency?

7.	What is the relationship between the location of the roots and stability?

8.	State the Routh Hurwitz criterion.

9.	What is the condition for marginal stability?

10.	What are the advantages of state space analysis?

PART – B  (5 x 3 = 15 MARKS)

11.	Write the analogues between voltage and current electrical systems.

12.	The damping ratio of a system is 0.75 and the natural frequency of oscillation is 12 rad/sec. Determine the peak time.

13.	What is breakaway and breaking point?

14.	Determine the stability of the given system using Routh criterion:

G(s)H(s)=9s2(s+2).

15.	Write the Eigen values for the given matrix.

010302−12−7−6

PART – C  (5 x 15 = 75 MARKS)

16.	Find the transfer function of the system given below by block diagram reduction method.

(OR)

[P.T.O]

17.	For the block diagram given above construct the signal flow graph and find the transfer function using Mason’s Gain formula.

18.	The open loop transfer function of a unity feedback system is given G(s) = K/s(sT+1) where K and T are positive constants. By what factor should the amplifier gain K be reduced, so that the peak overshoot of unit step response of the system is reduced from 75% to 25%?

(OR)

19.	A system is described by d2ydt2+8dydt+25y(t)=50x(t). Evaluate the response and maximum output for a step of 2.5 units.

20.	Sketch the bode plot for the given transfer function and determine the system gain K for the 	gain cross over frequency of 5 rad/sec. GS=Ks2(1+0.2s)(2s+1).

(OR)

21.	The open loop transfer function of a unity feedback system is given by G(s)=1s(s+1)(2s+1).

Sketch the polar plot and determine the gain margin and phase margin.

22.	Sketch the root locus for the system having:

G(s)=ks+1   and H(s)=(s+1)s2+4s+5.

(OR)

23.	A system is given by s5+2s4 +24s3+48s2–25s–50=0.  Check the stability of the system using Routh criterion and also find the frequency of oscillation.

24.	A system is described by

x1'

Check the controllability and observability of the system.

(OR)

25.	Determine the state equation and output equation of a system whose state space representation is

x1x2x3=010001−6−11−6 x1x2x3+006u

y=100 x1x2x3