Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MECHANICS OF MACHINES – II						      	       Time: 3 hours

Subject Code: 10ME201			       				               	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	When the crank is at the inner dead centre, in a horizontal reciprocating steam engine, then the velocity of the piston will be_________________.

2. 	In an engine, the work done by inertia forces in a cycle is _______________.
3. 	The maximum fluctuation of energy is the _____________.

4. 	The ratio of the maximum fluctuation of speed to the mean speed is called_____________.

5. 	The balancing of rotating and reciprocating parts of an engine is necessary when it runs at_________________.

6. 	The primary unbalanced force is maximum when the angle of inclination of the crank with the line of stroke is _______________.

7. 	When there is a reduction in amplitude over every cycle of vibration, then the body is said to have__________________.

8. 	The factor which affects the critical speed of a shaft is ______________.

9. 	At a nodal point in a shaft, the amplitude of torsional vibration is _______________.

10. 	A shaft carrying two rotors as its ends will have_________________.

PART – B  (5 x 3 = 15 MARKS)

11.	State the D-Alembert’s principle

12.	State the function of a flywheel and how it differs from a governor.

13.	State the expression for hammer blow for an uncoupled two cylinder locomotive engine.

14.	Give the expression for time period of longitudinal vibrations.

15 	State the assumptions made in determining the free torsional vibrations of a geared system.

PART – C  (5 x 15 = 75 MARKS)

16. 	The crank and connecting rod of a steam engine are 0.3 m and 1.5 m in length. The crank rotates at 180 rpm clockwise. Determine the velocity and acceleration of the piston when the crank is at 400 from the inner dead centre position. Also determine the position of the crank for zero acceleration of the piston.

(OR)

17.	A vertical petrol engine of 100 mm diameter and 120 mm stroke has a connecting rod of 250 mm length. The mass of the piston is 1.1 kg. The speed is 2000 rpm. On the expansion stroke with a crank 200 from TDC, the gas pressure is 700 kN/m2. Determine the net force on the piston, resultant load on the gudgeon pin and thrust on the cylinder walls.

18.	The turning moment diagram for a multi cylinder engine has been drawn to a scale                     of 1 mm = 600N-m vertically and 1 mm = 3 degree. The intercepted areas between the output torque curve and the mean resistance line, taken in order from one end are as follows: +52,              –24, +92,-140, +85, –72 and +107 mm2, when the engine is running at a speed of 600 rpm. If the total fluctuation of speed is not to exceed ± 1.5% of the mean, find the necessary mass of the flywheel of radius 0.5 m.

[P.T.O]

(OR)

19.	The turning moment curve for an engine is represented by the equation T=(20000+9500sin2θ−5700cos2θ)N-m where Ɵ is the angle moved by the crank from the IDC. If the resisting torque is constant, determine the power developed by the engine and the moment of inertia of the flywheel in kg-m2. The total fluctuation of the speed is not to exceed 1 % of the mean speed which is 180 rpm and the angular acceleration of the flywheel when the crank has turned through 450 from IDC.

20.	A, B, C and D are four masses carried by a rotating shaft at radii 100, 125, 200 and 150 mm respectively. The planes in which the masses revolve are spaced 600 mm apart and the masses of B , C and D are 10 kg, 5 kg and 4 kg respectively. Find the required mass A and the relative angular settings of the four masses so that the shaft shall be in complete balance.

(OR)

21.	A single cylinder reciprocating engine has speed 240 r.p.m., stroke 300 mm, mass of reciprocating parts 50 kg, mass of revolving parts at 150 mm radius, 37 kg. If two-third of the reciprocating parts and all the revolving parts are to be balanced, find:

a.	The balance mass required at a radius of 400 mm.

b. 	The residual unbalanced force when the crank has rotated 60° from top dead centre.

22.	The following data are given for a vibratory system with viscous damping: Mass = 2.5 kg; spring constant = 3 N/mm and the amplitude decreases to 0.25 of the initial value after five consecutive cycles. Determine the damping coefficient of the damper in the system.

(OR)

23.	The measurements on a mechanical vibrating system show that it has a mass of 8 kg and that the springs can be combined to give an equivalent spring of stiffness 5.4 in N/mm. If the vibrating system has a dashpot attached which exerts a force of 40 N when the mass has a velocity of 1 m/s, find:

a.	critical damping coefficient		b. 	damping factor

c.	logarithmic decrement			d.	 ratio of two consecutive amplitudes

24.	A steel shaft 1.5 m long is 95 mm in diameter for the first 0.6 m of its length, 60 mm in 	diameter for the next 0.5 m of length and 50 mm in diameter for the remaining 0.4 m of its 	length. The shaft carries two flywheels at two ends, the first having a mass of 900 kg and 0.85 	m radius of gyration located at the 95 mm diameter end and the second having a mass of 700 	kg and 0.55 m radius of gyration located at the other end. Determine the location of the node 	and the natural frequency of the torsional vibration of the system. The modulus of rigidity of 	shaft material may be taken as 80 GN/m2.

(OR)

25.	Determine the natural frequencies of torsional oscillation for the following system. The system is a reciprocating I. C. engine coupled to a centrifugal pump through a pair of gears. The shaft from the flywheel of the engine to the gear wheel is 60 mm diameter and 950 mm length. The shaft from the pinion to the pump is of 40 mm diameter and 300 mm length. The engine speed is 1/4 th of the pump speed.

Moment of inertia of the flywheel 		= 800 kg-m2

Moment of inertia of the gear wheel	= 15 kg-m2

Moment of inertia of the pinion		= 4 kg-m2

Moment of inertia of the pump 		= 17 kg-m2

Modulus of rigidity for the shaft material is 84 GN/m2