Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: FINITE ELEMENT ANALYSIS							       Time: 3 hours

Subject Code: 10ME205				       				              	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	In Galerkin’s method, the error ϵ(x) is called as _______________.

2.	The total potential energy Π of an elastic body is defined as the sum of _______ and _________.

3.	Define flexibility co-efficient.

4.	Give stiffness matrix of a simple beam element.

5.	Why is a convergence criterion very important in finite element analysis?

6.	What is meant by stability analysis?

7.	Write down the shape function of cubic element.

8.	When is an element called as isoparametric element?

9.	What is the characteristic matrix of 1D potential flow element?

10.	State the principles of virtual energy.

PART – B (5 x 3 = 15 MARKS)

11.	What is meant by Discretization and Assemblage?

12.	What is the difference between static and dynamic analysis?

13.	Define CST and LST.

14.	What is the purpose of an Isoparametric element?

15.	What is the difference between explicit and implicit solution of an assembled matrix?

PART – C (5 x 15 = 75 MARKS)

16.	Find the expression for the maximum deflection and maximum bending moment using        Raleigh-Ritz Method for a simply supported beam of length L subjected to uniformly distributed load q.

(OR)

17.	Explain in detail the various procedures for the formulation of the finite element problem.

18.	For a tapered plate of uniform thickness t=10mm, find the displacement at the nodes by                 forming into two element model. The bar has mass density ρ=7800 kg/m3, Young’s modulus, E=2*105MN/m2. In addition to self-weight, the plate is subjected to the point load P=10kN at its center.

(OR)

19.	Consider the bar elements. Area of element 1, A1=300 mm2 Area of element 2, A2= 300 mm2, Area of element 3, A3=600 mm2, Length of element 1, L1=200mm, Length of element 2, L2=200mm, Length of element 3, L3=400mm, E = 2*105 N/mm2, Point Load p = 400kN =400 * 103 N. Calculate The nodal displacement at point P,

[P.T.O]

20.	A thin plate of uniform thickness 25mm is subjected to a point load of 25N at mid depth, the plate is also subjected to self-weight. If E=2*105 N/mm2, ρ=.8*10-4 N/mm3 calculate, Nodal displacement.

(OR)

21.	Derive the stiffness matrix for a constant strain triangular element.

22.	A bridge is supported by several concrete piles and the geometry and loads of typical piles. The load 20 KN/m2 represents weight of the bridge and an assumed distribution of the traffic on the bridge. The concrete weight approximately 25 KN/m3 and its modulus f is 28 x106 KN/m2. Analyze the pile for stress and displacement by FEM having more than one element.

(OR)

23.	Derive the shape function for a six node triangular element.

24.	Solve the given problem for temperature distribution. Consider the end convection coefficient. Take T1=1700 C, h1= 0.0025 w/ cm2 0C, h2= 0.0625 w/ cm2 0C,  Tα=250C, l=10 cm, k= 0.17 w/ cm0C.

(OR)

25.	Explain the basic theory about bending behavior of plates and how it can be solved by FEA.