# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: ADVANCED STRENGTH OF MATERIALS     				 Time: 3 hours

# Subject Code: 10ME301				         		        		 	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

A Continuous beam ABC covers two consecutive spans AB and BC of lengths 4 m and 6 m carrying a UDL of 6 kN/m respectively. If the ends A and C are simply supported, find the support moments at A and C.

2.	Explain the Von Mises theory and compare it with the St. Venant theory.

(OR)

3.	State and explain the theories of failure.

4.  	Enumerate the typical design problems involving the theories with an axial loading.

(OR)

5. 	Explain briefly about the Castigliano’s first and second theorem.

6.  	Explain about the Rayleigh’s method applied to beams in bending.

(OR)

7. 	Calculate the maximum stress induced in a cast iron pipe of external diameter 40 mm and  internal diameter 20mm and length 4m when the pipe is supported at its ends and carries a point load of 80N at its centre.

8. 	Determine the shear stress function x and the value of the maximum shear stress for the equilateral triangle shown in the figure below. Each side consists of  length 2a√3.

(OR)

9. 	Derive the Airy stress function for the cantilever beam with the help of a partial differential stress equation.