Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: FINITE ELEMENT ANALYSIS							       Time: 3 hours

Subject Code: 10ME302				         				              	       Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

For a one dimensional composite bar shown in the figure below, determine the interface temperatures. For element l, let Kxx = 200W/(moC); for element 2, let  Kxx =100 W/(moC); and for element 3, let Kxx = 50W/(moC); Let A = 0.1m2. The left end has a constant temperature of 100oC and the right end has a constant temperature of 300oC.

2.	Find the second and third eigen values of the matrix:

[H] =	2−10−12−10−12

once X1  = 1.0−1.41421.0and λ=3.4142  are known.

(OR)

3.	Explain the finite difference method of solving propagation problems.

4.	Use the Rayleigh-Ritz method to find the displacement of the midpoint of the rod shown in the figure below.

Body force per unit volume,ρg=1.

[P.T.O]

(OR)

5.	A uniform rod subjected to a uniform axial load is illustrated in the figure below. The deformation of the bar is governed by the differential equation given below. Determine the displacement using weighted residual method.

6.	A tapered vertical rod of length 400 mm has diameters 200 mm and 100mm at its top and bottom ends respectively. The rod is fixed at the top end and subjected to a point load of 5 kN at its free end and a surface traction load of  2kN/m  in addition to its self-weight. Take Young’s modulus as 210 GPa and density as 7800 kg/m3. Model the plate with two spar elements and determine the nodal displacements and stresses in the elements.

(OR)

7.	Determine the natural frequencies and mode shapes of transverse vibration for a beam fixed at both ends. The beam may be modeled by two elements, each of length L and cross-sectional area A. Consider lumped mass matrix approach.

8.	Derive the body force vector and traction force vector (considering UDL acting on one of the edges of the element) for CST element.

(OR)

9.	Explain the finite element formulation towards the solution of the Poisson's equation in the context of torsion of prismatic shafts.