Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: AIRCRAFT STRUCTURES								       Time: 3 hours

Subject Code: 11AE202				       				              	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Which one of the following is a statically indeterminate structure?

a. Truss		b. Cantilever beam		c. Fixed beam			d. Over hanging beam

2. 	Which of the following criteria is for a redundant frame?

a. m= 2j-3	b. m < 3j-2			c. m > 3j-2		d. m >2j-3

3. 	Euler’s formula is used to find the buckling load of ___________ column.

a. long		b. medium		c. short 		d. all of the three

4. 	For a given column of length ‘L’  if both the ends are fixed then the characteristic length of the column is _____________.

a. L		b. L/√2		c. L/2			d. 2L

5.	To analyze the continuous beam which of the following methods cannot be used?

a. Unit load method						b. Clapeyron’s theorem of three moment

c. Moment distribution method				d. Energy method

6. 	The total strain energy stored in a body is commonly known as ______________.

a. resilience		b. proof resilience	c. modulus of resilience	d. modulus of elasticity

7. 	According to Bredt Batho’s formula, torque produced in a multi-cell closed cell is given by

a. 2A/q		b. 2q/A		c. 2q1/A1 + 2q2/A2  		d. 2A1q1+2A2q2

8. 	Shear center coincides with the centroid of the given shell when the shell is ___________.

a. unsymmetric	b. one axis of symmetry	c. two axes of symmetry	d. none of the above

9. 	In a _______ wing, the wing bending moments decrease rapidly spanwise from the maximum values at the fuselage support points.

a. cantilever		b. semi-cantilever		c. rigidly fixed		d. all of the above

10. 	When a compressive load is applied to a sheet stringer combination, the thin sheet buckles at _____________.

a. yield stress	      b. ultimate stress		c. normal stress		d. low stress

PART – B (5 x 3 = 15 MARKS)

11. 	Differentiate between statically determinate and statically indeterminate structure.

12. 	Why is the unit load method used?

13. 	What is Southwell’s plot?

14. 	Define Elastic axis.

15. 	What is a Wagner beam?

PART – C (5 x 15 = 75 MARKS)

16. 	Determine the forces in the members of the truss shown in the figure.

[P.T.O]

(OR)

17. 	State and prove Clapeyron’s theorem of three moment equation.

18. 	Analyze the continuous beam shown in the figure by strain energy method and determine the reactions.

(OR)

19. 	State and prove Castigliano’s theorem.

20. 	A solid round bar 60mm in diameter and 2.5m long is used as a strut. One end of the strut is fixed, while 	its other end is hinged. Find the safe compressive load for this strut using Euler’s formula. Assume E= 	200 GN/m2 and F.O.S.= 3. Derive the formula used.

(OR)

21. 	Derive the formula used for columns subjected to eccentric loading using Euler’s method.

22.  	Calculate the shear flow and shear center for the given section shown in figure.

(OR)

23.  	Find the shear flow and twist per unit length for the given section shown in the figure given that  Torque = 75000 N-cm. The material used is Aluminum with Young’s modulus E = 70 GPa and Poisson ratio = 0.3. Both the cells are square in cross section.

24. 	a.	What are the various structural components contained in an aircraft? What are their functions? (8)

b.	Explain the construction of shear force and bending moment diagrams for a cantilever type of aircraft wing.							 										   (7)

(OR)

25. 	a.	Explain the pure and semi tension field beams.							 			   (8)

b.	Differentiate between the monocoque and semi monocoque type fuselage construction.		   (7)